# NVIDIA ISAAC SIM COMPLETE PROJECT HANDOVER

Kiểm toán toàn bộ mã nguồn và bản đồ quyền điều khiển để tích hợp Jetson

| Mục | Giá trị |
| --- | --- |
| Project root đã xác nhận | /home/thien/Desktop/He_Thong_Cat_Ni_3D |
| Ngày kiểm toán | 23/10/2026 |
| Chế độ | Chỉ đọc repository; không chạy script ghi scene; không sửa Isaac Sim |
| Stage mặc định thực tế | scenes/fabric_feed_cutting_world.usda |
| Kết luận chính | Hệ thống hiện là playback USD/USDC đã bake. ROS 2 chỉ hoàn chỉnh cho demo đèn; chưa có interface điều khiển máy cắt từ Jetson. |

Đối tượng đọc: kỹ sư Isaac Sim, robotics/ROS 2, Jetson và kỹ sư tiếp nhận dự án. Tài liệu này phân biệt bằng chứng trong code/scene với giả định thiết kế và đề xuất tích hợp.

# Tóm tắt điều hành

Dự án chứa ba hướng mô phỏng máy cắt và một demo đèn Jetson. Launcher hiện ưu tiên mô hình cấp vải hai bánh vi sai với nỉ compliant. Chuyển động máy cắt không do một controller runtime đóng vòng điều khiển; phần lớn đã được tính offline rồi ghi thành time samples trong USDA/USDC. Vì vậy, Jetson không được ghi trực tiếp lên các xform đang có time samples nếu chưa có cơ chế phân quyền và chuyển chế độ.

Stage mặc định: scenes/fabric_feed_cutting_world.usda, giống hoàn toàn scenes/fabric_feed_compliant_candidate.usda theo SHA-256.

Scene mặc định đã đổi namespace cơ khí sang /World/AirTable, nhưng nhiều relationship và toàn bộ script bake vẫn dùng /World/FeedHead hoặc /World/CuttingHead. Đây là lỗi nhất quán path có bằng chứng trực tiếp trong USDA.

Camera là camera render thật, nhưng nhận diện gân, confidence và kiểm tra vision đều là dữ liệu tổng hợp/bake. Không có pipeline pixel hoặc RGB-D khép kín.

Mô hình nỉ chính là cache XPBD đã bake và reference từ compliant_felt_cache.usdc; không phải PhysX live end-to-end. Thử nghiệm PhysX nằm ở scene riêng.

ROS 2 hiện chỉ có demo đèn: Float32 /sensor/light và Bool /light/cmd. Không có ROS 2 topic/action/service cho trục máy cắt, trạng thái cắt, camera hoặc safety.

Nhiều controller đèn có thể ghi cùng /World/Lamp/Light.inputs:intensity. Chỉ hai live loop nội bộ có logic hủy nhau; ROS bridge và script one-shot không có arbitration.

Ba core dump tổng cộng khoảng 25,7 GB không đọc được do quyền root; metadata Git không hợp lệ vì .git chỉ là thư mục rỗng.

## Kết luận phân quyền Jetson

| Miền | Quyền nên áp dụng |
| --- | --- |
| Máy cắt playback hiện tại | Isaac Sim giữ quyền ghi time samples; Jetson chỉ nhận feedback hoặc gửi setpoint vào adapter tương lai, không ghi Prim trực tiếp. |
| Planner hình học và state machine | Không triển khai trùng trên Jetson ở giai đoạn đầu. Nếu Jetson là controller thật, chỉ chọn một nguồn sự thật và chuyển planner hiện có thành dịch vụ/setpoint source qua interface được thiết kế riêng. |
| Camera và vision | Isaac Sim gửi ảnh/timestamp/calibration; Jetson có thể đảm nhiệm perception thật. Không coi custom vision attributes đã bake là measurement live. |
| Đèn demo | Jetson có thể sở hữu quyết định ON/OFF, còn Isaac bridge là writer duy nhất của intensity. Phải tắt các controller nội bộ cùng lúc. |
| Safety | Không giao Jetson quyền trực tiếp trước khi có watchdog, heartbeat, command timeout, mode/ownership handshake và interlock độc lập. |

# Mục lục tài liệu

Chương 1 Tổng quan dự án

Chương 2 Thông tin môi trường

Chương 3 Toàn bộ cấu trúc thư mục

Chương 4 Phân tích chi tiết từng file

Chương 5 Phân tích USD Stage

Chương 6 Cấu tạo và chức năng các bộ phận máy

Chương 7 Kiến trúc và tương tác hệ thống

Chương 8 Camera cảm biến và xử lý dữ liệu

Chương 9 Điều khiển và mô phỏng

Chương 10 Jetson và ROS 2

Chương 11 Lỗi xung đột và rủi ro

Chương 12 Tiến độ phát triển

Chương 13 Hướng dẫn chuyển sang PC mới

Chương 14 Hướng dẫn Codex tiếp tục dự án

Phụ lục A đến H

# Chương 1 Tổng quan dự án

Mục tiêu hiện thực trong repository là nghiên cứu digital twin cho xén mép phôi nỉ 3D. Dự án phát triển qua ba kiến trúc: gantry XYZC, đầu dao cố định với bàn mang bốn bánh, và đầu dao cố định với cấp vải hai bánh vi sai. Subsystem đèn Jetson là một demo độc lập để thử Python, cảm biến ảo và ROS 2; nó không điều khiển máy cắt.

| Subsystem | Scene | Mục đích | Trạng thái |
| --- | --- | --- | --- |
| Cấp vải compliant mặc định | fabric_feed_cutting_world.usda | Hai bánh stepper, hai roller dưới, roller nắn, dao Z, nỉ XPBD cache và seam release. | Playback đã bake; validation có ngoại lệ; path namespace không nhất quán. |
| Fixed head bốn bánh | fixed_head_cutting_world.usda | Bàn mang phôi 4 bánh độc lập đưa đường cắt qua dao cố định. | Kinematic reference; validation PASS; không cắt vật liệu thật. |
| Gantry XYZC cũ | cutting_world.usda | Y bridge, X carriage, Z slide, C head theo đường tâm hai gân. | Kinematic reference; planner tổng hợp; không fracture. |
| PhysX thử nghiệm | physx_felt_test.usda và physx_felt_only_test.usda | Kiểm tra deformable surface và joint ổn định. | Scene thử riêng; chưa thay thế cache XPBD chính. |
| Jetson light | world.usda và jetson_light_world.usda | Phòng, đèn, sensor ảo và ROS 2 ON/OFF. | Code có đủ hai node; chưa có bằng chứng nghiệm thu Jetson thật. |

# Chương 2 Thông tin môi trường

| Thành phần | Bằng chứng | Kết luận |
| --- | --- | --- |
| Isaac Sim | scripts/container_init.sh dòng 17; docs/PHYSX_FELT_SETUP.md | Isaac Sim 5.1 trong Docker, Ubuntu 24.04. |
| Docker image | ISAAC_SIM.md dòng 3 và start_isaac.sh dòng 8 | ghrc_2026:v0; image phải tồn tại trên máy mới. |
| ROS 2 | scripts/container_init.sh dòng 17 đến 25 | ROS 2 Jazzy nội bộ Isaac Sim; RMW mặc định rmw_fastrtps_cpp. |
| GPU | start_isaac.sh dòng 110 đến 112 | Launcher yêu cầu --gpus all, IPC host và shared memory mặc định 8 GB. |
| Python | Không có pin phiên bản ở repository | Python của /isaac-sim/python.sh là nguồn runtime. Bytecode cho 3.11 và 3.14 tồn tại nhưng không đủ để kết luận runtime chuẩn. |
| Packages | Imports trong source | pxr, omni, isaacsim, numpy, scipy, Pillow, rclpy, std_msgs. Không có requirements/lockfile. |
| Biến môi trường | start_isaac.sh dòng 8 đến 11 và 125 đến 129 | IMAGE_NAME, CONTAINER_NAME, SHM_SIZE, AIC_ISAAC_STAGE, ROS_DOMAIN_ID, ROS_AUTOMATIC_DISCOVERY_RANGE, RMW_IMPLEMENTATION. |

Launcher mount toàn bộ project read-write vào /workspace/He_Thong_Cat_Ni_3D và tạo cache trong .isaac_cache. Vì vậy, chạy launcher hoặc script tạo scene không phải thao tác chỉ đọc. Kiểm toán này không chạy chúng.

# Chương 3 Toàn bộ cấu trúc thư mục

He_Thong_Cat_Ni_3D/
├── scripts/                 launcher, tạo scene, validation, UI, ROS 2
│   ├── cutting/             chuỗi phát triển máy cắt và PhysX
│   └── jetson_light/        demo cảm biến/đèn/ROS 2
├── src/cutting/             planner và solver không phụ thuộc UI Isaac
├── scenes/                  scene hiện hành, backup, USD cache và Blender
├── output/                  CSV, JSON, render và validation đã sinh
├── docs/                    tài liệu kỹ thuật theo từng nhánh
├── artifacts/phan_tich_anh/ ảnh tham chiếu và sơ đồ vùng khó
├── isaac_projects/          bản sao cũ của demo đèn
├── .isaac_cache/            cache runtime lớn, có thư mục không đủ quyền đọc
├── core.1 core.62 core.63   core dump không đủ quyền đọc
├── start_isaac.sh           launcher Docker
└── README.md ISAAC_SIM.md   hướng dẫn nhưng có chỗ lỗi thời

| Loại | Số file |
| --- | --- |
| .1 | 1 |
| .62 | 1 |
| .63 | 1 |
| .blend | 1 |
| .blend1 | 1 |
| .csv | 6 |
| .jpg | 1 |
| .json | 17 |
| .log | 1 |
| .md | 7 |
| .png | 33 |
| .py | 52 |
| .pyc | 60 |
| .sh | 2 |
| .svg | 1 |
| .txt | 1 |
| .usda | 21 |
| .usdc | 3 |
| [none] | 2 |

Ngoài .isaac_cache, kiểm kê được 212 file. .isaac_cache có ít nhất ba nhánh bị Permission denied nên không thể khẳng định đã đọc toàn bộ cache. Cache và bytecode được ghi nhận nhưng không sao chép mã bên thứ ba.

# Chương 4 Phân tích chi tiết từng file

Bảng dưới đây lập hồ sơ cho toàn bộ file Python và shell. Các script đánh số 01 đến 31 là pipeline tạo/chỉnh scene theo thời điểm phát triển; chúng không tự động cấu thành một runtime duy nhất.

| File | Vai trò | Class Function và đối tượng chính | Dependencies |
| --- | --- | --- | --- |
| artifacts/phan_tich_anh/render_phan_vung.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | polygon dòng 19, callout dòng 24. Prim literal: không có | pathlib, PIL |
| isaac_projects/scripts/jetson_light/01_create_structure.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | resolve_stage_path dòng 16, create_stage dòng 64, main dòng 97. Prim literal: /World, /World/Lamp, /World/Lamp/LampBody, /World/Lamp/Light, /World/Room, /World/Room/Floor, /World/Room/Wall_Back, /World/Room/Wall_Left, /World/Room/Wall_Right, /World/Sensors, /World/Sensors/LightSensor, /World/Table | pathlib, sys, pxr |
| isaac_projects/scripts/jetson_light/02_create_basic_geometry.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | resolve_stage_path dòng 25, require_prim dòng 43, set_transform dòng 51, set_display_color dòng 62, create_cube dòng 67, create_sphere dòng 84, create_sphere_light dòng 101, build_geometry dòng 119, main dòng 200. Prim literal: /World/Lamp/LampBody, /World/Lamp/Light, /World/Room/Floor, /World/Room/Wall_Back, /World/Room/Wall_Left, /World/Room/Wall_Right, /World/Sensors/LightSensor, /World/Table | pathlib, sys, pxr |
| isaac_projects/scripts/jetson_light/03_test_scene.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | resolve_stage_path dòng 33, main dòng 59. Prim literal: /World/Lamp/LampBody, /World/Lamp/Light, /World/Room/Floor, /World/Room/Wall_Back, /World/Room/Wall_Left, /World/Room/Wall_Right, /World/Sensors/LightSensor, /World/Table | pathlib, sys, pxr |
| scripts/02_create_prim_tree.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | print_prim_tree dòng 27, create_stage dòng 42, main dòng 56. Prim literal: /World, /World/Lamp, /World/Lamp/LampBody, /World/Lamp/Light, /World/Room, /World/Room/Floor, /World/Room/Wall_Back, /World/Room/Wall_Left, /World/Room/Wall_Right, /World/Sensors, /World/Sensors/LightSensor, /World/Table | pathlib, pxr |
| scripts/03_create_basic_geometry.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | require_prim dòng 12, set_transform dòng 18, set_display_color dòng 45, create_cube dòng 50, create_sphere dòng 66, build_scene dòng 82, main dòng 160. Prim literal: /World/Lamp/LampBody, /World/Lamp/Light, /World/Room/Floor, /World/Room/Wall_Back, /World/Room/Wall_Left, /World/Room/Wall_Right, /World/Sensors/LightSensor, /World/Table | pathlib, pxr |
| scripts/container_init.sh | Launcher/entrypoint shell. | Không định nghĩa Python; đọc trực tiếp toàn file. | Docker, bash, Isaac Sim image và biến môi trường. |
| scripts/cutting/01_create_cutting_scene.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | _set_transform dòng 33, _material dòng 53, _bind dòng 72, _cube dòng 76, _cylinder dòng 91, _sphere dòng 109, _hexagonal_rotary_blade dòng 123, _raised_ridge_mesh dòng 162, _curve dòng 208, _height_field_slab dòng 227, _create_visual_holes dòng 304, _unwrap_headings dòng 332, _lookahead_index dòng 341, _animate_cutting_head dòng 357, _create_machine dòng 491, _create_scene dòng 601, _write_outputs dòng 716, main dòng 757. Prim literal: /World, /World/CuttingSystem, /World/Lighting/Dome, /World/Lighting/InspectionLight, /World/Looks, /World/Looks/{name}, /World/Looks/{name}/Shader, /World/Machine, /World/Machine/Frame, /World/Machine/Frame/YRail{side}, /World/Machine/Frame/{side}{end}Post, /World/Machine/Kinematics … | __future__, csv, json, math, pathlib, pxr, src.cutting.path_planner |
| scripts/cutting/02_validate_cutting_scene.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | _require dòng 47, main dòng 52. Prim literal: /World, /World/CuttingSystem, /World/Looks/Felt, /World/Machine/Frame/YRailLeft, /World/Machine/Frame/YRailRight, /World/Machine/Kinematics/YBridge, /World/Machine/Kinematics/YBridge/CrossBeam, /World/Machine/Kinematics/YBridge/LeftYCarriage, /World/Machine/Kinematics/YBridge/LeftYServo, /World/Machine/Kinematics/YBridge/RightYCarriage, /World/Machine/Kinematics/YBridge/RightYServo, /World/Machine/Kinematics/YBridge/XCarriage … | __future__, csv, json, math, pathlib, pxr |
| scripts/cutting/03_play_cutting_demo.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | play_cutting_demo dòng 13, pause_cutting_demo dòng 19, reset_cutting_demo dòng 24. Prim literal: không có | omni.timeline |
| scripts/cutting/05_create_fixed_head_scene.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | attr dòng 30, cube dòng 36, cyl dòng 40, xf dòng 44, material dòng 50, label dòng 58, make_blade dòng 62, make_table dòng 80, make_carrier dòng 103, make_tool dòng 162, make_vision dòng 191, main dòng 241. Prim literal: /World, /World/Carrier, /World/Cell/Floor, /World/Cell/RimX{side+1}, /World/Cell/RimY{side+1}, /World/Cell/TableLeg{i}, /World/Cell/TableTop, /World/Controller, /World/FixedHead/CameraBracket, /World/FixedHead/CameraBracketCrossTie, /World/FixedHead/FixedBridge, /World/FixedHead/Foot{side+1} … | __future__, csv, importlib, json, math, pathlib, pxr, src.cutting.fixed_head_planner, src.cutting.path_planner |
| scripts/cutting/06_validate_fixed_head_scene.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | main dòng 14. Prim literal: /World/Carrier, /World/Carrier/Wheels/Module{i+1}, /World/Carrier/Workpiece/, /World/FixedHead/InspectionCamera/Sensor, /World/FixedHead/ZSlide, /World/Looks/Felt, /World/Machine/Kinematics/YBridge | csv, json, math, pathlib, pxr, src.cutting.fixed_head_planner |
| scripts/cutting/07_fixed_head_controls.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | FixedHeadControls dòng 10. Prim literal: /World/Controller, /World/FixedHead/InspectionCamera/Sensor | omni.kit.app, omni.timeline, omni.ui, omni.usd, omni.kit.viewport.utility, pxr |
| scripts/cutting/08_render_fixed_head.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | capture dòng 28, main dòng 44. Prim literal: /World/Controller, /World/FixedHead/InspectionCamera/Sensor | pathlib, os, sys, traceback, isaacsim, numpy, PIL, omni.replicator.core, omni.timeline, omni.usd, isaacsim.core.utils.stage, pxr, carb |
| scripts/cutting/09_create_fabric_feed_scene.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | attr dòng 27, material dòng 33, cube dòng 41, cyl dòng 45, xform dòng 49, flat_ridge dòng 55, make_cell dòng 83, make_sheet dòng 106, make_omni_wheels dòng 124, make_knife dòng 173, make_vision dòng 200, main dòng 239. Prim literal: /World, /World/AirTable, /World/Cell/Floor, /World/Cell/Leg{i+1}, /World/Cell/Perforations/P_{ix}_{iy}, /World/Cell/SacrificialTop, /World/Cell/Table, /World/Controller, /World/CuttingHead/ZSlide, /World/FeedHead/FixedWheelFrameX{side+1}, /World/FeedHead/FixedWheelFrameY{side+1}, /World/FeedHead/KnifeCenterMount … | __future__, csv, importlib, json, math, pathlib, pxr, src.cutting.fabric_feed_planner, src.cutting.fixed_head_planner |
| scripts/cutting/10_validate_fabric_feed_scene.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | main dòng 17. Prim literal: /World/Carrier, /World/CuttingHead/ZSlide, /World/FeedHead/MainSupportArm, /World/FeedHead/OmniWheel{i}, /World/FeedHead/OmniWheel{i}/Rotor, /World/Frame/Post{i}, /World/LooseFabric, /World/LooseFabric/HoldDowns, /World/LooseFabric/ShapedNest, /World/Vision/InspectionCamera/Sensor, /World/Vision/LED{i}/Support | csv, json, math, pathlib, pxr, src.cutting.fabric_feed_planner |
| scripts/cutting/11_fabric_feed_controls.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | FabricFeedControls dòng 12. Prim literal: /World/Controller, /World/LooseFabric/Offcut, /World/Vision/InspectionCamera/Sensor | omni.kit.app, omni.timeline, omni.ui, omni.usd, omni.kit.viewport.utility, pxr |
| scripts/cutting/12_render_fabric_feed.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | capture dòng 23, main dòng 36. Prim literal: /World/Controller, /World/FeedHead/LowerPinchRoller, /World/LooseFabric, /World/Vision/InspectionCamera/Sensor | pathlib, json, os, traceback, isaacsim, numpy, PIL, omni.replicator.core, omni.timeline, omni.usd, isaacsim.core.utils.stage, pxr |
| scripts/cutting/13_convert_to_two_stepper_wheels.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | custom dòng 32, make_wheel dòng 40, main dòng 103. Prim literal: /World/Controller, /World/FeedHead/FixedWheelFrameY0, /World/FeedHead/FrontWheelLeft, /World/FeedHead/FrontWheelRight, /World/FeedHead/OmniWheel{i}, /World/Looks/, /World/LooseFabric | __future__, csv, importlib, json, math, pathlib, pxr, src.cutting.fabric_feed_planner |
| scripts/cutting/14_validate_two_stepper_wheels.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | main dòng 16. Prim literal: /World/Controller, /World/FeedHead/, /World/FeedHead/FixedWheelFrameY0, /World/FeedHead/OmniWheel{i} | csv, json, math, pathlib, pxr, src.cutting.fabric_feed_planner |
| scripts/cutting/15_upgrade_physical_cutting.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | custom dòng 36, bind dòng 47, rotate_xy dòng 51, fabric_surface_z dòng 56, update_deck_and_access_slots dòng 70, create_heightfield dòng 87, signed_area dòng 121, simplify_polygon dòng 126, point_in_triangle dòng 144, triangulate dòng 150, create_finished_part dòng 168, inside_polygon dòng 186, offset_polygon dòng 195, create_scrap dòng 210, update_surface_details dòng 248, local_surface_at_world dòng 263, make_spring dòng 268, make_pair dòng 296, main dòng 373. Prim literal: /World/AirTable/PinchAccessSlot{side}, /World/Cell/SacrificialTop, /World/Controller, /World/CuttingHead/ZSlide, /World/FeedHead/FrontWheelLeft, /World/FeedHead/FrontWheelRight, /World/FeedHead/FrontWheel{side}, /World/FeedHead/LowerPinchRollerLeft, /World/FeedHead/LowerPinchRollerRight, /World/FeedHead/LowerPinchRoller{side}, /World/FeedHead/SuspensionLeft, /World/FeedHead/SuspensionRight … | __future__, csv, importlib, json, math, pathlib, pxr, src.cutting.fixed_head_planner, src.cutting.fabric_feed_planner |
| scripts/cutting/16_validate_physical_cutting.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | close dòng 25, main dòng 29. Prim literal: /World/AirTable/PinchAccessSlot{side}, /World/Cell/SacrificialTop, /World/Controller, /World/FeedHead/FrontWheel{side}, /World/FeedHead/LowerPinchRoller{side}, /World/FeedHead/Suspension{side}, /World/LooseFabric, /World/LooseFabric/Blank, /World/LooseFabric/FinishedPart, /World/LooseFabric/SeparatedScrap | __future__, csv, json, math, collections, pathlib, pxr |
| scripts/cutting/17_rebuild_compliant_felt.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | attr dòng 30, clear_animation dòng 36, translate dòng 43, fixed_body dòng 48, spring dòng 53, smooth_cylinder dòng 63, triangular_gusset dòng 98, pinch_contact_skin dòng 116, mechanism dòng 155, metal_guide_tip dòng 275, straightening_rollers dòng 328, mesh_prim dòng 454, texture_material dòng 473, main dòng 504. Prim literal: /World/AirTable/PinchAccessSlot, /World/Cell/SacrificialTop, /World/Controller, /World/CuttingHead/ZSlide, /World/FeedHead/, /World/FeedHead/FixedWheelFrameY2, /World/FeedHead/FrontWheel, /World/FeedHead/HoldRollerFrame, /World/FeedHead/LowerPinchRoller, /World/FeedHead/OffcutChute, /World/FeedHead/OffcutSplitter, /World/FeedHead/PinchContactSkin … | pathlib, importlib, json, math, shutil, time, numpy, pxr, PIL, src.cutting.felt_shell, src.cutting.fixed_head_planner, src.cutting.fixed_head_planner, src.cutting.path_planner |
| scripts/cutting/18_render_compliant_felt.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | main dòng 19. Prim literal: /World/, /World/Looks/ReferenceFelt, /World/LooseFabric | os, json, traceback, pathlib, isaacsim, numpy, PIL, omni.usd, omni.timeline, omni.replicator.core, isaacsim.core.utils.stage, pxr |
| scripts/cutting/19_check_shell.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | Không định nghĩa class/function cấp module. Prim literal: không có | numpy, pxr, pathlib, src.cutting.felt_shell |
| scripts/cutting/20_validate_compliant_felt.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | visible_triangles dòng 47, row dòng 67, exact_rows dòng 74, prominence dòng 77, components dòng 426. Prim literal: /World/Controller, /World/CuttingHead/ZSlide, /World/FeedHead/FixedWheelFrameY2, /World/FeedHead/FrontWheel, /World/FeedHead/FrontWheelLeft/Rotor, /World/FeedHead/FrontWheelRight/Rotor, /World/FeedHead/HoldRoller, /World/FeedHead/HoldRollerFrame, /World/FeedHead/HoldSuspension, /World/FeedHead/LowerPinchRoller, /World/FeedHead/PinchContactSkin, /World/FeedHead/StraighteningRoller … | json, os, sys, pathlib, numpy, pxr, scipy.sparse, scipy.sparse.csgraph, src.cutting.felt_shell, scipy.spatial, pxr |
| scripts/cutting/21_sync_cloth_cache_clock.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | Không định nghĩa class/function cấp module. Prim literal: không có | pathlib, pxr |
| scripts/cutting/22_finalize_candidate.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | set_attr dòng 12. Prim literal: /World/Controller, /World/LooseFabric | pathlib, pxr |
| scripts/cutting/24_fill_large_table_openings.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | set_string dòng 18, restore_small_perforations dòng 25, make_continuous_deck dòng 47, main dòng 82. Prim literal: /World/AirTable, /World/AirTable/PinchAccessSlotLeft, /World/AirTable/PinchAccessSlotRight, /World/Cell/Perforations, /World/Cell/SacrificialTop, /World/FeedHead/OffcutChute, /World/FeedHead/OffcutSplitter | pathlib, shutil, pxr |
| scripts/cutting/25_compact_direct_wheel_mount.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | set_attr dòng 26, set_translate dòng 33, set_cube dòng 43, define_cube dòng 51, set_cylinder dòng 64, define_smooth_cylinder dòng 73, spring_points dòng 111, main dòng 123. Prim literal: /World/Controller, /World/FeedHead/FixedWheelFrameY2, /World/FeedHead/FrontWheel{side}, /World/FeedHead/LowerPinchRoller{side}, /World/FeedHead/Suspension{side}, /World/Looks/Rubber, /World/Looks/Steel | pathlib, math, shutil, numpy, pxr |
| scripts/cutting/27_render_pinch_diagnostic.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | capture dòng 20. Prim literal: /World/FeedHead/FrontWheel, /World/FeedHead/LowerPinchRoller | pathlib, os, isaacsim, numpy, PIL, omni.replicator.core, omni.usd, isaacsim.core.utils.stage, pxr |
| scripts/cutting/28_scrub_candidate_animation.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | main dòng 9. Prim literal: /World/Controller, /World/CuttingHead/ZSlide, /World/FeedHead/FrontWheel{side}/Rotor, /World/FeedHead/LowerPinchRoller{side}/Rotor | pathlib, pxr |
| scripts/cutting/29_add_pinch_contact_skins.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | main dòng 13. Prim literal: /World/FeedHead/PinchContactSkin{side}, /World/Looks/ReferenceFelt | pathlib, importlib, pxr, src.cutting.felt_shell |
| scripts/cutting/30_apply_pending_mechanism_layout.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | main dòng 15. Prim literal: /World/Controller | pathlib, importlib, shutil, pxr |
| scripts/cutting/30_physx_felt_setup.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | layer_signature dòng 89, _stable dòng 103, changed_outside dòng 118, mesh_area dòng 130, world_scale dòng 145, shape_volume dòng 151, bound_look_name dòng 186, Scope dòng 195, ensure_physics_material dòng 221, bind_physics_material dòng 238, configure_collider dòng 245, configure_tools dòng 260, felt_photo_albedo dòng 292, felt_normal_map dòng 316, configure_felt_pbr dòng 328, create_felt_test_sheet dòng 378, configure_felt dòng 433, body_inertia_about_axis dòng 492, configure_joints dòng 514, create_physics_scene dòng 580, configure dòng 595, main dòng 621. Prim literal: /World, /World/FeedHead/FrontWheelLeft, /World/PhysicsScene | __future__, argparse, math, shutil, pathlib, sys |
| scripts/cutting/31_physx_felt_smoke_test.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | world_points dòng 47, world_yaw dòng 53. Prim literal: không có | __future__, argparse, json, os, sys, isaacsim, numpy, omni.timeline, omni.usd, isaacsim.core.utils.stage, pxr |
| scripts/cutting/test_path_planner.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | main dòng 17. Prim literal: không có | __future__, math, pathlib, sys, src.cutting.path_planner |
| scripts/jetson_light/01_create_structure.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | create_stage dòng 51, main dòng 75. Prim literal: /World, /World/Lamp, /World/Lamp/LampBody, /World/Lamp/Light, /World/Room, /World/Room/Floor, /World/Room/Wall_Back, /World/Room/Wall_Left, /World/Room/Wall_Right, /World/Sensors, /World/Sensors/LightSensor, /World/Table | pathlib, pxr |
| scripts/jetson_light/02_create_basic_geometry.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | require_prim dòng 28, set_transform dòng 36, set_display_color dòng 63, create_cube dòng 68, create_sphere dòng 85, create_sphere_light dòng 102, build_geometry dòng 120, main dòng 201. Prim literal: /World/Lamp/LampBody, /World/Lamp/Light, /World/Room/Floor, /World/Room/Wall_Back, /World/Room/Wall_Left, /World/Room/Wall_Right, /World/Sensors/LightSensor, /World/Table | pathlib, pxr |
| scripts/jetson_light/03_test_scene.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | main dòng 50. Prim literal: /World/Lamp/LampBody, /World/Lamp/Light, /World/Room/Floor, /World/Room/Wall_Back, /World/Room/Wall_Left, /World/Room/Wall_Right, /World/Sensors/LightSensor, /World/Table | pathlib, sys, pxr |
| scripts/jetson_light/04_test_light.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | read_intensity dòng 18, turn_light_on dòng 25, turn_light_off dòng 30, main dòng 35. Prim literal: /World/Lamp/Light | pathlib, pxr |
| scripts/jetson_light/05_virtual_light_sensor.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | parse_args dòng 15, main dòng 26. Prim literal: /World/Sensors/LightSensor | argparse, pathlib, pxr |
| scripts/jetson_light/06_automatic_light_control.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | parse_args dòng 20, decide_light_state dòng 31, main dòng 36. Prim literal: /World/Lamp/Light, /World/Sensors/LightSensor | argparse, pathlib, pxr |
| scripts/jetson_light/06_daylight_sensor_simulation.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | _get_or_create_float_attr dòng 45, _get_or_create_sun dòng 53, _daylight_from_hour dòng 73, _update_sensor_and_lights dòng 80, _daylight_loop dòng 135, set_simulated_time dòng 168, set_day_duration dòng 186, pause_daylight_cycle dòng 198, resume_daylight_cycle dòng 206, stop_daylight_sensor_simulation dòng 214, start_daylight_sensor_simulation dòng 223. Prim literal: /World/Environment, /World/Environment/Sun, /World/Lamp/Light, /World/Sensors/LightSensor | asyncio, builtins, math, time, omni.kit.app, omni.usd, pxr |
| scripts/jetson_light/06_live_automatic_light_control.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | set_virtual_light_level dòng 23, _live_control_loop dòng 52, stop_live_light_controller dòng 102, start_live_light_controller dòng 111. Prim literal: /World/Lamp/Light, /World/Sensors/LightSensor | asyncio, builtins, omni.kit.app, omni.usd, pxr |
| scripts/jetson_light/07_isaac_ros2_light_bridge.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | parse_args dòng 26, IsaacLightBridge dòng 37, main dòng 85. Prim literal: /World/Lamp/Light, /World/Sensors/LightSensor | argparse, time, pathlib, rclpy, rclpy.node, std_msgs.msg, pxr |
| scripts/jetson_light/08_jetson_light_controller.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | parse_args dòng 20, decide_light_command dòng 32, JetsonLightController dòng 37, main dòng 62. Prim literal: không có | argparse, time, rclpy, rclpy.node, std_msgs.msg |
| scripts/open_stage.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | load_project_stage dòng 27. Prim literal: không có | asyncio, os, pathlib, omni.kit.app, omni.kit.actions.core, omni.usd |
| src/cutting/__init__.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | Không định nghĩa class/function cấp module. Prim literal: không có | path_planner |
| src/cutting/fabric_feed_planner.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | FabricSample dòng 30, drive_direction dòng 47, _mapped_z dòng 57, plan_cycle dòng 76, validate_plan dòng 122. Prim literal: không có | __future__, dataclasses, math, src.cutting.fixed_head_planner |
| src/cutting/felt_shell.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | contour dòng 96, holes dòng 113, inside dòng 126, outward_normals dòng 134, miter_distance dòng 141, rib_profile dòng 171, base_shape_z dòng 184, shape_z dòng 199, swept_contact_points dòng 205, build_mesh dòng 231, ShellSolver dòng 343. Prim literal: không có | __future__, math, numpy, scipy.spatial, scipy.sparse |
| src/cutting/fixed_head_planner.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | PathPoint dòng 29, Sample dòng 36, rotate dòng 53, unwrap dòng 58, outline dòng 62, heading dòng 102, ridge_paths dòng 106, _smooth dòng 122, plan_cycle dòng 126, body_displacement dòng 209, _wheel_commands dòng 217, validate_plan dòng 281. Prim literal: không có | __future__, dataclasses, math, src.cutting.path_planner |
| src/cutting/path_planner.py | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. | PlannerConfig dòng 16, CutPoint dòng 28, _catmull_rom dòng 76, sample_closed_spline dòng 101, surface_height dòng 121, _signed_area dòng 130, _unit_tangent dòng 138, offset_closed_path dòng 149, _curvature dòng 165, _confidence dòng 178, _speed_for_point dòng 190, _zone dòng 202, plan_cut_path dòng 210, closed_path_length dòng 290. Prim literal: không có | __future__, dataclasses, math, typing |
| start_isaac.sh | Launcher/entrypoint shell. | Không định nghĩa Python; đọc trực tiếp toàn file. | Docker, bash, Isaac Sim image và biến môi trường. |

## Phân nhóm sử dụng thực tế

Được launcher gọi tự động: scripts/open_stage.py, sau đó scripts/cutting/11_fabric_feed_controls.py cho stage mặc định.

Nguồn tạo scene mặc định: chuỗi 09, 13, 15, 17, 21, 22, 24, 25 và các script layout/scrub liên quan; không có manifest tái dựng chính thức.

Runtime playback: time samples trong USDA/USDC và omni.timeline. Planner không được gọi lại mỗi frame.

Validation độc lập: 02, 06, 10, 14, 16, 20, 31 và test_path_planner.py. PASS chỉ xác nhận invariant mà test mã hóa.

Bản sao isaac_projects là nhánh demo đèn cũ, dùng cơ chế resolve path khác và không được launcher chính import.

# Chương 5 Phân tích USD Stage

| Stage | Dung lượng | Default Prim | m per unit | Up | TCPS | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| isaac_projects/scenes/jetson_light_world.usda | 2.8 KiB | "World" | không khai báo | "Z" | không khai báo | Không có |
| scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda | 935.8 KiB | "World" | 1 | "Z" | 60 | ../output/compliant_felt/felt_fibres.png; ./compliant_felt_cache.usdc</Cloth> |
| scenes/cutting_world.usda | 553.6 KiB | "World" | 1 | "Z" | 60 | Không có |
| scenes/cutting_world_user_edit_backup.usda | 340.6 KiB | "World" | 1 | "Z" | 60 | Không có |
| scenes/fabric_feed_before_compliant_promotion.usda | 11674.4 KiB | "World" | 1 | "Z" | 60 | Không có |
| scenes/fabric_feed_before_compliant_v2.usda | 11674.4 KiB | "World" | 1 | "Z" | 60 | Không có |
| scenes/fabric_feed_before_straightening_rollers.usda | 1835.9 KiB | "World" | 1 | "Z" | 60 | ../output/compliant_felt/felt_fibres.png; ./compliant_felt_cache.usdc</Cloth> |
| scenes/fabric_feed_compliant_candidate.usda | 2018.2 KiB | "World" | 1 | "Z" | 60 | ../output/compliant_felt/felt_fibres.png; ./compliant_felt_cache.usdc</Cloth> |
| scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda | 896.9 KiB | "World" | 1 | "Z" | 60 | ../output/compliant_felt/felt_fibres.png; ./compliant_felt_cache.usdc</Cloth> |
| scenes/fabric_feed_cutting_world.before_fill_large_openings.usda | 811.5 KiB | "World" | 1 | "Z" | 60 | ../output/compliant_felt/felt_fibres.png; ./compliant_felt_cache.usdc</Cloth> |
| scenes/fabric_feed_cutting_world.before_hold_rollers.usda | 1198.6 KiB | "World" | 1 | "Z" | 60 | ../output/compliant_felt/felt_fibres.png; ./compliant_felt_cache.before_hold_rollers.usdc</Cloth> |
| scenes/fabric_feed_cutting_world.before_inboard_wheel_spacing.usda | 899.0 KiB | "World" | 1 | "Z" | 60 | ../output/compliant_felt/felt_fibres.png; ./compliant_felt_cache.usdc</Cloth> |
| scenes/fabric_feed_cutting_world.before_physical_upgrade.usda | 3257.6 KiB | "World" | 1 | "Z" | 60 | Không có |
| scenes/fabric_feed_cutting_world.before_remove_table_perforations.usda | 900.5 KiB | "World" | 1 | "Z" | 60 | ../output/compliant_felt/felt_fibres.png; ./compliant_felt_cache.usdc</Cloth> |
| scenes/fabric_feed_cutting_world.before_ridge_fix.usda | 935.8 KiB | "World" | 1 | "Z" | 60 | ../output/compliant_felt/felt_fibres.png; ./compliant_felt_cache.hold_v3.usdc</Cloth> |
| scenes/fabric_feed_cutting_world.usda | 2018.2 KiB | "World" | 1 | "Z" | 60 | ../output/compliant_felt/felt_fibres.png; ./compliant_felt_cache.usdc</Cloth> |
| scenes/fixed_head_cutting_world.usda | 4814.9 KiB | "World" | 1 | "Z" | 60 | Không có |
| scenes/jetson_light_world.usda | 2.8 KiB | "World" | không khai báo | "Z" | không khai báo | Không có |
| scenes/physx_felt_only_test.usda | 1869.0 KiB | "World" | 1 | "Z" | 60 | ../output/compliant_felt/felt_fibres.png; ./compliant_felt_cache.hold_v3.usdc</Cloth>; ../output/compliant_felt/felt_photo_albedo.png; ../output/compliant_felt/felt_photo_normal.png |
| scenes/physx_felt_test.usda | 1899.7 KiB | "World" | 1 | "Z" | 60 | ../output/compliant_felt/felt_fibres.png; ./compliant_felt_cache.hold_v3.usdc</Cloth>; ../output/compliant_felt/felt_fibres.png; ../output/compliant_felt/felt_fibres_normal.png |
| scenes/world.usda | 5.7 KiB | "World" | 1 | "Z" | không khai báo | Không có |

## Stage mặc định và hierarchy chức năng

/World
├── SacrificialTop
├── AirTable
│   ├── FixedWheelFrameY2
│   ├── SuspensionLeft Right
│   │   └── StraightenerMount StraightenerSteeringJoint
│   ├── FrontWheelLeft Right Rotor
│   ├── LowerPinchRollerLeft Right Rotor
│   ├── StraighteningRollerLeft Right Rotor SwivelHead FabricGuideBar
│   ├── ZSlide Blade
│   ├── InspectionCamera Sensor
│   ├── LED1 Light  LED2 Light
│   └── Ambient Overhead
├── Controller
└── LooseFabric
    ├── FinishedPart  composed from compliant_felt_cache.usdc /Cloth
    └── Offcut        composed from compliant_felt_cache.usdc /Cloth

Metadata xác nhận metersPerUnit = 1, upAxis = Z, timeCodesPerSecond = 60 và endTimeCode khoảng 12.380. Reference duy nhất của scene chính là ./compliant_felt_cache.usdc</Cloth>. File cache là USD crate 0.8.0, khoảng 1,45 GB; không thể đọc bằng parser text, nhưng code tạo cache và validation xác nhận hai mesh FinishedPart/Offcut có points time-sampled.

## Physics và composition

Scene chính chứa RigidBodyAPI kinematic trên frame, suspension, bánh và roller. Chuyển động vẫn là time samples, không phải torque/velocity drive live.

Hai StraightenerSteeringJoint có axis Z, giới hạn ±12° và targetPosition 0. Đồng thời body mang xformOp:rotateZ time-sampled và kinematicEnabled = true; drive không phải nguồn chuyển động thực tế trong playback.

Các relationship physics:body0/body1 trong scene chính vẫn trỏ /World/FeedHead/... dù Prim đã nằm dưới /World/AirTable/.... Đây là target dangling và làm mất ý nghĩa joint nếu chuyển sang dynamic.

Không tìm thấy /World/PhysicsScene trong scene playback chính. PhysicsScene chỉ có trong physx_felt_test/only_test theo nhánh thử nghiệm.

Scene thử PhysX tạo deformable surface cho PhysxFeltTestSheet. Cache XPBD chính không được gán deformable live vì mesh hai mặt/time-sampled không phù hợp.

## Biến thể scene

| Scene | Chuyển động chính |
| --- | --- |
| cutting_world.usda | YBridge translate Y, XCarriage translate X, ZSlide translate Z, CHead rotate Z, BladeRotor rotate Y và custom vision attributes. |
| fixed_head_cutting_world.usda | Carrier translate XY và rotate Z, ZSlide translate Z, 4 steering rotate Z, 4 rotor rotate Y, controller state. |
| fabric_feed_cutting_world.usda | Bánh/roller rotate Y, straightener translate/steer/spin, ZSlide Z, controller state và cloth points qua reference. |
| world.usda | Không timeline; light intensity và sensor custom attributes bị script runtime ghi. |

# Chương 6 Cấu tạo và chức năng các bộ phận máy

| Bộ phận | Prim path | Cơ chế và trạng thái |
| --- | --- | --- |
| Bàn và SacrificialTop | /World/SacrificialTop; /World/AirTable | Mặt đỡ và khung cơ khí. Không chuyển động trong playback. |
| Hai bánh kéo trên | /World/AirTable/FrontWheelLeft và Right | Bánh cao su R22 mm, trục quay Y, stepper chỉ là metadata/hình học; góc rotor đã bake. |
| Hai roller dưới | /World/AirTable/LowerPinchRollerLeft và Right | Roller bị động R14 mm, trục Y; time samples quay theo bánh trên. |
| Hai roller nắn thẳng | /World/AirTable/StraighteningRollerLeft và Right | Lăn Y, steering Z giới hạn ±12°, hướng đã bake từ vận tốc vải. |
| Dao và trượt Z | /World/AirTable/ZSlide/Blade | Dao cố định XY, chỉ nâng/hạ Z trong scene mặc định. |
| Camera | /World/AirTable/InspectionCamera/Sensor | Camera render; không có module inference pixel live. |
| LED | /World/AirTable/LED1/Light; LED2/Light | Chiếu sáng cố định intensity 1800. |
| Vải compliant | /World/LooseFabric/FinishedPart; Offcut | Mesh cache XPBD; điểm vertex thay đổi theo thời gian, seam release mô phỏng cắt. |
| Controller metadata | /World/Controller | State, corner, oscillation flag, released welds và mô tả giới hạn. |

# Chương 7 Kiến trúc và tương tác hệ thống

Sơ đồ kiến trúc hệ thống

flowchart LR
  Launcher[start_isaac.sh] --> Open[open_stage.py]
  Open --> Stage[fabric_feed_cutting_world.usda]
  Open --> UI[FabricFeedControls]
  Planner[fixed_head_planner plan_cycle] --> Bake[17_rebuild_compliant_felt]
  Shell[felt_shell ShellSolver] --> Bake
  Bake --> Cache[compliant_felt_cache.usdc]
  Cache --> Stage
  Stage --> Timeline[Isaac timeline playback]
  Timeline --> View[Viewport and camera]
  Jetson[Jetson] -. no cutting interface .-> Stage

Sơ đồ phụ thuộc module

flowchart TD
  path_planner --> fixed_head_planner
  fixed_head_planner --> fabric_feed_planner
  fixed_head_planner --> rebuild_compliant_felt
  felt_shell --> rebuild_compliant_felt
  rebuild_compliant_felt --> USDA
  rebuild_compliant_felt --> USDC
  open_stage --> controls
  controls --> timeline

Luồng dữ liệu

flowchart LR
  SyntheticGeometry --> plan_cycle
  plan_cycle --> Frames[pose state wheel commands]
  Frames --> ShellSolver
  ShellSolver --> VertexCache
  Frames --> USDTimeSamples
  VertexCache --> Playback
  USDTimeSamples --> Playback
  Playback --> CameraRender
  CameraRender -. no inference .-> Planner

Trình tự runtime

sequenceDiagram
  participant D as Docker launcher
  participant O as open_stage.py
  participant U as USD Stage
  participant C as UI Controls
  participant T as Timeline
  D->>O: exec after Isaac startup
  O->>U: open_stage_async default stage
  O->>C: exec FabricFeedControls
  C->>T: Play or seek
  T->>U: evaluate baked time samples
  U-->>C: state corner releasedWelds
  C-->>User: status and viewport

# Chương 8 Camera cảm biến và xử lý dữ liệu

| Cảm biến | Prim | Dữ liệu | Producer | Consumer | Trạng thái |
| --- | --- | --- | --- | --- | --- |
| Inspection camera mặc định | /World/AirTable/InspectionCamera/Sensor | RGB render | Isaac renderer | Viewport/render scripts | Camera thật trong scene; không inference. |
| Custom vision gantry | .../LineCamera/Sensor | detectedInner/Outer/Center XYZ, confidence, valid | 01_create_cutting_scene._animate_cutting_head | USD playback/validation | Synthetic time samples, không từ pixel. |
| Fixed head camera | /World/InspectionCamera/Sensor | RGB render | Isaac renderer | 08_render_fixed_head và viewport | Render đã có artefact; planner vẫn dùng hình học. |
| Light sensor | /World/Sensors/LightSensor | light_level, daylight_level, time_of_day | manual, daylight loop hoặc script 05 | auto light và ROS publisher | Sensor ảo; không timestamp/header. |

Không tìm thấy depth camera, IMU, contact sensor, TF, camera_info, frame_id hoặc cơ chế đồng bộ RGB/depth. ROS message Float32 không có timestamp. Vì vậy, Jetson vision cần data contract mới; chưa được phép tự tạo trong phạm vi báo cáo này.

# Chương 9 Điều khiển và mô phỏng

Bảng dưới đây là bản đồ quyền điều khiển bắt buộc. “Writer lịch sử” nghĩa là script đã tạo time samples trong file; khi playback, USD timeline là nguồn áp dụng giá trị. “Writer runtime” nghĩa là code có thể ghi trực tiếp khi đang chạy.

| Prim Bộ phận | Controller hiện tại | File | Input | Output | Quyền ghi | Nguy cơ xung đột Jetson | Khuyến nghị |
| --- | --- | --- | --- | --- | --- | --- | --- |
| /World/AirTable/FrontWheelLeft/Right/Rotor — bánh kéo | USD time samples; nguồn lịch sử 17_rebuild_compliant_felt.main dòng 652–655 | scripts/cutting/17_rebuild_compliant_felt.py | pose và vận tốc frame từ fixed_head_planner.plan_cycle; ShellSolver | xformOp:rotateY, trục Y | USD/Isaac ghi khi timeline chạy | Cao: path code cũ là /World/FeedHead; ghi live sẽ đấu với time samples | Chỉ nhận setpoint Jetson qua adapter có mode ownership; không ghi Prim trực tiếp. |
| /World/AirTable/LowerPinchRollerLeft/Right/Rotor — roller dưới | USD time samples; cùng loop bake dòng 655 | scripts/cutting/17_rebuild_compliant_felt.py | góc bánh trên và tỉ số bán kính | xformOp:rotateY, trục Y | USD/Isaac | Cao nếu Jetson điều khiển độc lập; roller là passive về cơ khí | Giữ Isaac điều khiển hoặc feedback-only; Jetson không cấp lệnh riêng. |
| /World/AirTable/StraighteningRollerLeft/Right — càng nắn | USD time samples; main dòng 630–638 | scripts/cutting/17_rebuild_compliant_felt.py | vector vận tốc vải từ pose planner | xformOp:rotateZ và translate Z; steering Z | USD/Isaac | Cao: joint drive targetPosition 0 đồng tồn tại; body kinematic | Adapter trung gian. Chọn hoặc xform kinematic hoặc joint drive, không dùng đồng thời. |
| /World/AirTable/StraighteningRollerLeft/Right/Rotor | USD time samples; main dòng 657 | scripts/cutting/17_rebuild_compliant_felt.py | spin bánh kéo và tỉ số bán kính | xformOp:rotateY; trục Y | USD/Isaac | Trung bình; roller được mô tả passive | Feedback-only hoặc Isaac playback. |
| /World/AirTable/SuspensionLeft/Right/.../StraightenerSteeringJoint | DriveAPI targetPosition = 0; configure_joints có thể chỉnh stiffness/damping | 17_rebuild_compliant_felt.py; 30_physx_felt_setup.py | target position, inertia tính từ geometry khi chạy config | drive:angular:physics:*; trục Z | USD/PhysX nếu body dynamic | Rất cao: body1 relationship trỏ path cũ và body đang kinematic/time-sampled | Chưa giao Jetson. Sửa namespace và xác định dynamic mode trước; cần adapter. |
| /World/AirTable/ZSlide — dao Z | USD time samples; nguồn lịch sử main dòng 645–650 | scripts/cutting/17_rebuild_compliant_felt.py | felt_mid, state CUT/CORNER_STOP, độ sâu dao | xformOp:translate Z | USD/Isaac | Rất cao và nguy hiểm nếu Jetson ghi khi timeline chạy | Chỉ nhận Z setpoint qua adapter có interlock; hiện giữ Isaac. |
| /World/LooseFabric/FinishedPart và Offcut — lưới nỉ | USDC points time samples từ ShellSolver | 17_rebuild_compliant_felt.py; src/cutting/felt_shell.py | pose, seam count, gravity, constraint/contact | points vertex XYZ; topology tĩnh | USD reference/cache | Không nên để Jetson ghi; dữ liệu rất lớn và là kết quả mô phỏng | Isaac-only; gửi feedback tóm tắt cho Jetson. |
| /World/Controller — state | USD time samples main dòng 640–644; UI chỉ đọc | 17_rebuild_compliant_felt.py; 11_fabric_feed_controls.py | frame state và progress | control:state cornerId oscillationEnabled cut:releasedWelds | USD/Isaac | Cao nếu Jetson ghi state song song; không có arbitration | Isaac giữ state playback. Jetson chỉ gửi requested mode/setpoint qua adapter. |
| /World/LooseFabric/Offcut — visibility | FabricFeedControls.play và toggle_offcut dòng 36–59 | scripts/cutting/11_fabric_feed_controls.py | nút UI | visibility | UI runtime | Thấp; chỉ quan sát, nhưng có thể che sai trạng thái cắt | Giữ Isaac UI; không đưa sang Jetson. |
| /World/Carrier — bàn mang 4 bánh scene fixed head | Time samples từ 05_create_fixed_head_scene.main dòng 279–296 | scripts/cutting/05_create_fixed_head_scene.py | fixed_head_planner.plan_cycle | translate XY, rotateZ | USD/Isaac | Cao nếu scene này được chọn và Jetson ghi pose | Chỉ setpoint qua adapter; không tái triển khai inverse kinematics trên Jetson nếu Isaac vẫn sở hữu. |
| /World/Carrier/.../Module1..4/Steering và Rotor | Time samples từ main; planner._wheel_commands | 05_create_fixed_head_scene.py; fixed_head_planner.py | carrier twist và vị trí bánh | steer rotateZ, spin rotateY, wheelSpeedMps | USD/Isaac | Cao; cùng trục có pose bake | Setpoint-only qua adapter hoặc feedback-only. |
| /World/ZSlide — dao Z scene fixed head | Time samples từ 05 main | scripts/cutting/05_create_fixed_head_scene.py | knife_tip_z từ planner state machine | translate Z | USD/Isaac | Rất cao về safety | Adapter/interlock bắt buộc. |
| /World/Machine/Kinematics/YBridge | _animate_cutting_head dòng 357 trở đi | scripts/cutting/01_create_cutting_scene.py | CutPoint y và time_s | translate Y | USD/Isaac | Cao nếu gantry scene chạy | Setpoint qua adapter; feedback pose về Jetson. |
| /World/.../XCarriage | _animate_cutting_head | scripts/cutting/01_create_cutting_scene.py | CutPoint x | translate X | USD/Isaac | Cao | Setpoint qua adapter. |
| /World/.../ZSlide và CHead | _animate_cutting_head | scripts/cutting/01_create_cutting_scene.py | z, heading, zone/confidence | translate Z; rotateZ C | USD/Isaac | Rất cao | Adapter và safety gate. |
| /World/.../BladeRotor | _animate_cutting_head | scripts/cutting/01_create_cutting_scene.py | closed path length / 0.060 | rotateY | USD/Isaac | Trung bình | Isaac-only visualization; không ánh xạ actuator thật. |
| /World/.../LineCamera/Sensor custom vision attrs | _animate_cutting_head | scripts/cutting/01_create_cutting_scene.py | synthetic lookahead CutPoint | detected XYZ confidence valid | USD/Isaac | Cao về nghĩa dữ liệu nếu Jetson coi là sensor thật | Feedback mô phỏng có provenance synthetic; không dùng làm measurement thật. |
| /World/Lamp/Light | 04 test; 06 one-shot; live loop; daylight loop; IsaacLightBridge.on_light_command | scripts/jetson_light/04, 06*, 07 | manual CLI, light_level hoặc ROS Bool | inputs:intensity | Nhiều writer Isaac và ROS | Rất cao: last writer wins; chỉ live/daylight tự hủy nhau | Chuyển quyết định cho Jetson; Isaac bridge là writer duy nhất. Tắt controller nội bộ. |
| /World/Sensors/LightSensor | 05 main; set_virtual_light_level; daylight loop | scripts/jetson_light/05, 06_live, 06_daylight | CLI/manual hoặc clock mô phỏng | light_level daylight_level time_of_day | Nhiều writer Isaac | Cao: Jetson nhận dữ liệu không biết nguồn và không timestamp | Isaac sở hữu sensor; gửi feedback có timestamp/source qua adapter tương lai. |
| /World/Environment/Sun | _get_or_create_sun và _update_sensor_and_lights | scripts/jetson_light/06_daylight_sensor_simulation.py | giờ mô phỏng | inputs:intensity và rotateY | Isaac live loop | Thấp nếu Jetson không điều khiển thời tiết | Giữ Isaac điều khiển; feedback tùy nhu cầu. |

## Chi tiết điều kiện kích hoạt và controller trùng

| Controller | Điều kiện kích hoạt | Controller cùng tác động |
| --- | --- | --- |
| USD playback máy cắt | Người dùng nhấn Play/restart hoặc timeline đang chạy. | Mọi writer live vào cùng attribute; scrub script nếu chạy ngoài runtime. |
| FabricFeedControls | Launcher tự exec khi tên stage là fabric_feed_cutting_world.usda; hành động theo nút UI/update event. | Timeline và user thao tác trực tiếp viewport/Property. |
| Live light controller | File được Execute; tự gọi start_live_light_controller; kiểm tra mỗi 15 frame. | Daylight loop, one-shot auto, test light, ROS bridge. |
| Daylight loop | Gọi start_daylight_sensor_simulation; cập nhật khoảng 10 Hz. | Live loop bị hủy, nhưng ROS bridge/one-shot không bị hủy. |
| ROS IsaacLightBridge | Node chạy và nhận Bool /light/cmd. | Mọi controller đèn nội bộ. |
| JetsonLightController | Nhận Float32 /sensor/light; publish Bool ngay mỗi message. | Logic threshold Isaac nội bộ nếu vẫn chạy. |

## Ba quyết định phải chốt trước tích hợp

Chọn một control owner tại mỗi thời điểm: USD playback, PhysX live hoặc Jetson external control.

Chốt namespace Prim canonical và sửa mọi relationship dangling trước khi bật joint dynamics hoặc nhận command ngoài.

Tách command, feedback và safety. Jetson không được ghi trực tiếp root layer hoặc attribute đang có time samples.

# Chương 10 Jetson và ROS 2

| Node | Publish | Subscribe | QoS | Tần suất | Ghi Stage |
| --- | --- | --- | --- | --- | --- |
| isaac_light_bridge | /sensor/light std_msgs/Float32 | /light/cmd std_msgs/Bool | depth 10; mặc định rclpy | Timer 1 Hz | inputs:intensity và Save root layer |
| jetson_light_controller | /light/cmd std_msgs/Bool | /sensor/light std_msgs/Float32 | depth 10; mặc định rclpy | Theo message | Không truy cập Stage |

Không có service, action, lifecycle, liveliness policy, deadline, durability cấu hình, TF, frame_id, timestamp, sequence number, command acknowledgement, heartbeat hoặc loss-of-connection handler. Node bridge mở file world.usda riêng và lưu root layer sau mỗi lệnh, không kết nối tới stage GUI đang mở. Đây là demo giao tiếp đèn, không phải Isaac ROS bridge cho máy cắt.

## A Chức năng Isaac Sim đã thực hiện

Tạo và phát lại ba mô hình chuyển động máy cắt; tính planner offline và ghi USD time samples.

Tạo cache biến dạng nỉ XPBD, seam release, camera/đèn và các artefact validation/render.

Tạo UI playback, seek góc, chuyển camera, ẩn/hiện offcut.

Mô phỏng cảm biến ánh sáng, ngày/đêm và ghi cường độ đèn.

Publish light_level và nhận lệnh đèn qua hai ROS 2 topic.

## B Chức năng Jetson không nên triển khai trùng

Không chạy lại cùng state machine và inverse kinematics trong khi USD playback vẫn sở hữu pose.

Không tự ghi Z slide, rotor, controller state hoặc cloth points trực tiếp.

Không chạy threshold đèn nội bộ Isaac đồng thời với JetsonLightController.

Không coi custom vision:detected* đã bake là đầu ra perception live rồi tính lại cùng logic.

## C Chức năng còn thiếu Jetson có thể đảm nhiệm

Perception RGB hoặc RGB-D thật, hiệu chuẩn camera-machine, phát hiện hai gân và confidence có provenance.

Giám sát sức khỏe, watchdog, command timeout và phát hiện mất kết nối.

Điều phối setpoint cấp cao khi hệ thống xác định Jetson là owner; không điều khiển trực tiếp Prim.

Thu thập telemetry, đồng bộ timestamp và lưu log liên thiết bị.

Tính toán vision/AI nếu cần; repository hiện không có AI.

## D Dữ liệu Isaac Sim cần gửi sang Jetson

RGB camera, camera_info/calibration, timestamp mô phỏng và frame id.

Pose X Y Z C hoặc carrier pose; wheel steer/spin/speed; trạng thái state machine.

cornerId, oscillationEnabled, releasedWelds, progress, simulation time và playback mode.

Sensor/physics feedback: joint state, contact/slip/traction nếu sau này có live physics; hiện nhiều dữ liệu chưa tồn tại.

Health, active controller owner, scene id/hash và cờ dữ liệu synthetic/baked/live.

## E Command và setpoint Jetson có thể gửi về Isaac Sim

Requested mode, start, pause, reset, e-stop request và heartbeat; phải qua adapter có xác nhận.

Setpoint X Y Z C hoặc carrier/wheel setpoint, nhưng chỉ khi Isaac chuyển sang external-control mode.

Vision result: paired ridge points 3D, confidence, timestamp và calibration version.

Light Bool hoặc intensity setpoint cho demo đèn; chỉ một bridge được quyền ghi intensity.

Không gửi cloth vertex points hoặc ghi trực tiếp controller state.

## F Điểm cần adapter hoặc interface nhưng không được tự tạo

Adapter namespace resolver giữa /World/FeedHead cũ và /World/AirTable hiện tại.

Control-mode arbiter giữa USD playback, PhysX drive và external Jetson setpoint.

ROS 2 interface cho machine command, machine state, vision, camera, heartbeat và fault.

Timestamp/clock adapter cho simulation time và Jetson wall/ROS time.

Safety adapter với Z interlock, knife enable, clamp/pinch verification và command timeout.

Scene/session-layer adapter để tránh ghi đè root USDA khi nhận lệnh runtime.

# Chương 11 Lỗi xung đột và rủi ro

| Mức | Vấn đề | Bằng chứng và điều kiện xảy ra | Đề xuất tương lai |
| --- | --- | --- | --- |
| Rất cao | Namespace hiện tại khác code | Scene có /World/AirTable nhưng script dùng /World/FeedHead /World/CuttingHead /World/Vision. Relationship joint vẫn trỏ path cũ. | Không chạy lại pipeline hoặc Jetson command cho tới khi có mapping được xác nhận. |
| Rất cao | Nhiều writer đèn | Ít nhất năm code path ghi inputs:intensity; không có owner token. | Chỉ một writer bridge; tắt các loop khác. |
| Rất cao | Z và dao không có safety interface | Z là time-sampled; chưa có e-stop/limit/ack/watchdog. | Không giao quyền Jetson trực tiếp. |
| Cao | USD root layer bị lưu từ ROS callback | IsaacLightBridge.on_light_command gọi Save mỗi message. | Tương lai dùng session layer hoặc command state riêng; báo cáo không sửa. |
| Cao | README mâu thuẫn | Đầu README nói launcher mở fixed head, nhưng open_stage.py và start_isaac.sh ưu tiên fabric feed. | Tin code launcher hiện tại. |
| Cao | Candidate và main giống hệt nhưng tài liệu nói candidate riêng | SHA-256 hai file bằng nhau. | Ghi rõ trạng thái promotion đã xảy ra, dù script 17 chỉ export candidate. |
| Cao | Cloth cache cực lớn | 1,45 GB và backup 1,15 GB; chuyển máy dễ thiếu LFS/asset. | Sao chép và xác minh checksum. |
| Cao | Core dump lớn | Ba file root-only khoảng 25,7 GB. | Không cần chuyển để chạy; điều tra crash riêng. |
| Trung bình | Metadata/test không phải runtime proof | PASS xác nhận invariant mô hình; không chứng minh thiết bị thật hoặc ma sát. | Giữ phân loại “đã kiểm thử mô hình” thay vì “đã nghiệm thu”. |
| Trung bình | Không có dependency lock | Không requirements/lockfile; phụ thuộc image cục bộ. | Mang đúng Docker image hoặc lập BOM sau này. |
| Trung bình | Git metadata không hợp lệ | .git chỉ là thư mục, git báo not a repository. | Không có lịch sử commit để truy vết. |

# Chương 12 Tiến độ phát triển

| Phân loại | Chức năng | Bằng chứng |
| --- | --- | --- |
| Đã triển khai và có bằng chứng kiểm thử | Planner đường cắt tổng hợp, kinematic gantry, fixed-head bốn bánh, fabric-feed bake, UI playback, validation JSON/ảnh. | Các output PASS và scene/time samples tồn tại. |
| Đã viết nhưng chưa xác minh end-to-end | Hai ROS 2 node đèn và controller Jetson. | Code hoàn chỉnh, README nói chưa nghiệm thu trên Jetson thật. |
| Đang phát triển | PhysX deformable và joint dynamics. | Scene thử/smoke test riêng; scene chính vẫn dùng XPBD cache. |
| Chưa triển khai | Vision khép kín từ pixel/depth, machine ROS interface, closed-loop traction, safety PLC/interlock, hardware I/O. | Nhiều metadata và tài liệu nêu rõ not implemented. |
| Không thể xác định | Tác vụ cuối cùng của tác giả và lịch sử commit. | .git không hợp lệ; timestamp cho thấy cập nhật đến 22/10/2026 nhưng không đủ suy ra ý định. |

# Chương 13 Hướng dẫn chuyển sang PC mới

Sao chép toàn bộ repository trừ khi chủ sở hữu quyết định bỏ core dumps và cache runtime. Bắt buộc giữ scenes/compliant_felt_cache.usdc và mọi texture output được USD tham chiếu.

Xác minh Docker image ghrc_2026:v0 hoặc một image tương đương Isaac Sim 5.1 có ROS 2 Jazzy. Repository không chứa Dockerfile để tái tạo image.

Cài NVIDIA driver/GPU tương thích Isaac Sim 5.1 và Docker GPU runtime. Launcher dùng --gpus all, --network host, --ipc host và shm 8 GB.

Giữ project mount trong container tại /workspace/He_Thong_Cat_Ni_3D hoặc sửa mọi absolute path/launcher một cách có chủ đích.

Đặt ROS_DOMAIN_ID, ROS_AUTOMATIC_DISCOVERY_RANGE và RMW_IMPLEMENTATION phù hợp mạng Jetson; không ghi secret vào repository.

Trước khi chạy GUI, kiểm tra checksum các scene lớn và reference ./compliant_felt_cache.usdc.

Chạy GUI bằng ./start_isaac.sh. Stage dự kiến là fabric_feed_cutting_world.usda. Xác nhận UI FabricFeedControls xuất hiện và timeline phát lại.

Chạy validation chỉ trên bản sao hoặc khi chấp nhận chúng có thể tạo output. Không chạy script tạo/nâng cấp scene như một bước kiểm tra read-only.

Với ROS đèn, bảo đảm rclpy/std_msgs từ Isaac ROS bridge và ROS 2 Jazzy trên Jetson; test topic discovery trước, sau đó chỉ bật một light writer.

## Asset bắt buộc và đường dẫn cần chú ý

| Asset | Vai trò | Rủi ro |
| --- | --- | --- |
| scenes/compliant_felt_cache.usdc | Cache points time-sampled cho FinishedPart/Offcut | 1,45 GB; thiếu file thì scene chính mất vải động. |
| output/compliant_felt/felt_fibres.png | Texture relative ../output/compliant_felt/felt_fibres.png | Phải giữ tương quan thư mục. |
| output/compliant_felt/felt_photo_albedo.png và normal.png | PBR PhysX/test | Được tạo bởi script 30 nếu chạy; hiện tồn tại. |
| scenes/felt_reference_sample.usdc | Mẫu reference render | Khoảng 9,8 MB. |
| scenes/he_thong_cat_ni.blend và .blend1 | Nguồn Blender nhị phân | Không thấy USD reference trực tiếp; vai trò nguồn hình học chưa xác định. |

# Chương 14 Hướng dẫn Codex tiếp tục dự án

Luôn đọc stage mặc định thực tế và kiểm tra Prim path trước khi dựa vào tài liệu hoặc script cũ.

Không chạy script đánh số theo chuỗi nếu người dùng chưa yêu cầu thay đổi. Nhiều script ghi đè scene, backup hoặc output.

Không coi USD time samples là controller hardware. Xác định rõ offline bake, playback, PhysX live và external control.

Nếu tích hợp Jetson, bắt đầu bằng data contract và ownership matrix; không ghi trực tiếp Prim khi timeline đang sở hữu attribute.

Giữ rõ nhãn synthetic, baked, assumed, provisional, uncalibrated và unverified trong mọi output.

Đối chiếu README với open_stage.py và start_isaac.sh; code launcher thắng khi mâu thuẫn.

Không sửa namespace bằng phỏng đoán. Trước hết lập mapping path và kiểm tra relationship target.

# Phụ lục A Danh sách toàn bộ file

| Đường dẫn | Bytes | Vai trò và phạm vi kiểm tra |
| --- | --- | --- |
| .gitignore | 34 | File hỗ trợ hoặc cấu hình. |
| ISAAC_SIM.md | 1,004 | Tài liệu dự án; dùng làm bằng chứng nhưng đối chiếu lại với code/scene. |
| README.md | 11,671 | Tài liệu dự án; dùng làm bằng chứng nhưng đối chiếu lại với code/scene. |
| artifacts/phan_tich_anh/mau_vai_ghe_goc.jpg | 570,276 | Tài nguyên hình ảnh/tham chiếu. |
| artifacts/phan_tich_anh/phan_vung_so_bo.png | 3,110,182 | Tài nguyên hình ảnh/tham chiếu. |
| artifacts/phan_tich_anh/phan_vung_so_bo.svg | 3,742 | Tài nguyên hình ảnh/tham chiếu. |
| artifacts/phan_tich_anh/render_phan_vung.py | 3,674 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| core.1 | 10,343,129,088 | Core dump nhị phân; không đọc được do quyền root; bằng chứng crash, không phải mã nguồn. |
| core.62 | 6,780,538,880 | Core dump nhị phân; không đọc được do quyền root; bằng chứng crash, không phải mã nguồn. |
| core.63 | 9,447,337,984 | Core dump nhị phân; không đọc được do quyền root; bằng chứng crash, không phải mã nguồn. |
| docs/COMPLIANT_FELT_V2.md | 12,740 | Tài liệu dự án; dùng làm bằng chứng nhưng đối chiếu lại với code/scene. |
| docs/CUTTING_SIMULATION.md | 5,946 | Tài liệu dự án; dùng làm bằng chứng nhưng đối chiếu lại với code/scene. |
| docs/FIXED_HEAD_CUTTING.md | 8,086 | Tài liệu dự án; dùng làm bằng chứng nhưng đối chiếu lại với code/scene. |
| docs/PHYSX_FELT_SETUP.md | 10,701 | Tài liệu dự án; dùng làm bằng chứng nhưng đối chiếu lại với code/scene. |
| isaac_projects/README.md | 4,385 | Tài liệu dự án; dùng làm bằng chứng nhưng đối chiếu lại với code/scene. |
| isaac_projects/scenes/jetson_light_world.usda | 2,816 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| isaac_projects/scripts/jetson_light/01_create_structure.py | 3,640 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| isaac_projects/scripts/jetson_light/02_create_basic_geometry.py | 8,391 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| isaac_projects/scripts/jetson_light/03_test_scene.py | 3,097 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| isaac_projects/scripts/jetson_light/__pycache__/01_create_structure.cpython-314.pyc | 4,769 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| isaac_projects/scripts/jetson_light/__pycache__/02_create_basic_geometry.cpython-314.pyc | 11,734 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| isaac_projects/scripts/jetson_light/__pycache__/03_test_scene.cpython-314.pyc | 4,341 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| output/compliant_felt/cut_early.png | 880,073 | Ảnh render/QA sinh ra. |
| output/compliant_felt/cut_finished.png | 872,194 | Ảnh render/QA sinh ra. |
| output/compliant_felt/cut_middle.png | 936,683 | Ảnh render/QA sinh ra. |
| output/compliant_felt/cut_progress.json | 392,196 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/compliant_felt/felt_fibres.png | 1,422,292 | Ảnh render/QA sinh ra. |
| output/compliant_felt/felt_fibres_normal.png | 2,202,895 | Ảnh render/QA sinh ra. |
| output/compliant_felt/felt_photo_albedo.png | 1,051,226 | Ảnh render/QA sinh ra. |
| output/compliant_felt/felt_photo_normal.png | 1,511,625 | Ảnh render/QA sinh ra. |
| output/compliant_felt/hold_cutaway.png | 472,411 | Ảnh render/QA sinh ra. |
| output/compliant_felt/hold_rollers.png | 733,691 | Ảnh render/QA sinh ra. |
| output/compliant_felt/mechanism.png | 614,118 | Ảnh render/QA sinh ra. |
| output/compliant_felt/nip_low_view.png | 570,149 | Ảnh render/QA sinh ra. |
| output/compliant_felt/nip_low_view_corner.png | 596,790 | Ảnh render/QA sinh ra. |
| output/compliant_felt/nip_top_view.png | 659,060 | Ảnh render/QA sinh ra. |
| output/compliant_felt/pinch_cutaway.png | 436,440 | Ảnh render/QA sinh ra. |
| output/compliant_felt/reference_shape.png | 699,943 | Ảnh render/QA sinh ra. |
| output/compliant_felt/reference_side.png | 364,168 | Ảnh render/QA sinh ra. |
| output/compliant_felt/render_report.json | 1,174 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/compliant_felt/simulation_report.json | 883 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/compliant_felt/validation.json | 4,035 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/cutting/line_camera_capture.log | 30 | Artefact đầu ra mô phỏng. |
| output/cutting/line_camera_frame.png | 129,414 | Ảnh render/QA sinh ra. |
| output/cutting/simulation_summary.json | 1,412 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/cutting/trim_path_plan.csv | 100,757 | Dữ liệu lệnh hoặc quỹ đạo xuất từ planner/bake. |
| output/fabric_feed/corner_events.json | 588 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/fabric_feed/finished_head_detail.png | 628,037 | Ảnh render/QA sinh ra. |
| output/fabric_feed/finished_separation.png | 316,778 | Ảnh render/QA sinh ra. |
| output/fabric_feed/front_alignment.png | 545,735 | Ảnh render/QA sinh ra. |
| output/fabric_feed/head_detail.png | 609,160 | Ảnh render/QA sinh ra. |
| output/fabric_feed/head_detail_steered.png | 630,797 | Ảnh render/QA sinh ra. |
| output/fabric_feed/machine_overview.png | 323,111 | Ảnh render/QA sinh ra. |
| output/fabric_feed/motion_commands.csv | 1,810,606 | Dữ liệu lệnh hoặc quỹ đạo xuất từ planner/bake. |
| output/fabric_feed/physical_upgrade_summary.json | 417 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/fabric_feed/physical_validation.json | 1,153 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/fabric_feed/pinch_stepper_commands.csv | 1,522,267 | Dữ liệu lệnh hoặc quỹ đạo xuất từ planner/bake. |
| output/fabric_feed/pinch_with_lower.png | 643,554 | Ảnh render/QA sinh ra. |
| output/fabric_feed/pinch_without_any_wheels.png | 753,175 | Ảnh render/QA sinh ra. |
| output/fabric_feed/pinch_without_lower.png | 683,511 | Ảnh render/QA sinh ra. |
| output/fabric_feed/render_validation.json | 1,181 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/fabric_feed/sensor_corner.png | 424,504 | Ảnh render/QA sinh ra. |
| output/fabric_feed/sensor_rgb.png | 358,902 | Ảnh render/QA sinh ra. |
| output/fabric_feed/summary.json | 673 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/fabric_feed/two_stepper_summary.json | 370 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/fabric_feed/two_stepper_validation.json | 494 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/fabric_feed/two_stepper_wheel_commands.csv | 980,738 | Dữ liệu lệnh hoặc quỹ đạo xuất từ planner/bake. |
| output/fabric_feed/validation.json | 627 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/fixed_head/corner_events.json | 588 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/fixed_head/head_detail.png | 320,402 | Ảnh render/QA sinh ra. |
| output/fixed_head/machine_overview.png | 248,468 | Ảnh render/QA sinh ra. |
| output/fixed_head/motion_commands.csv | 1,825,604 | Dữ liệu lệnh hoặc quỹ đạo xuất từ planner/bake. |
| output/fixed_head/reference_path.csv | 38,633 | Dữ liệu lệnh hoặc quỹ đạo xuất từ planner/bake. |
| output/fixed_head/render_error.txt | 35 | Artefact đầu ra mô phỏng. |
| output/fixed_head/render_validation.json | 645 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/fixed_head/sensor_corner.png | 217,020 | Ảnh render/QA sinh ra. |
| output/fixed_head/sensor_rgb.png | 198,873 | Ảnh render/QA sinh ra. |
| output/fixed_head/summary.json | 810 | Kết quả/biên bản validation hoặc mô phỏng. |
| output/fixed_head/validation.json | 361 | Kết quả/biên bản validation hoặc mô phỏng. |
| scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda | 958,256 | Bản sao scene lịch sử; không phải stage mặc định. |
| scenes/compliant_felt_cache.before_hold_rollers.usdc | 1,146,834,263 | Bản sao scene lịch sử; không phải stage mặc định. |
| scenes/compliant_felt_cache.usdc | 1,454,709,247 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scenes/cutting_world.usda | 566,937 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scenes/cutting_world_user_edit_backup.usda | 348,789 | Bản sao scene lịch sử; không phải stage mặc định. |
| scenes/fabric_feed_before_compliant_promotion.usda | 11,954,551 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scenes/fabric_feed_before_compliant_v2.usda | 11,954,587 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scenes/fabric_feed_before_straightening_rollers.usda | 1,879,954 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scenes/fabric_feed_compliant_candidate.usda | 2,066,624 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda | 918,388 | Bản sao scene lịch sử; không phải stage mặc định. |
| scenes/fabric_feed_cutting_world.before_fill_large_openings.usda | 830,959 | Bản sao scene lịch sử; không phải stage mặc định. |
| scenes/fabric_feed_cutting_world.before_hold_rollers.usda | 1,227,408 | Bản sao scene lịch sử; không phải stage mặc định. |
| scenes/fabric_feed_cutting_world.before_inboard_wheel_spacing.usda | 920,623 | Bản sao scene lịch sử; không phải stage mặc định. |
| scenes/fabric_feed_cutting_world.before_physical_upgrade.usda | 3,335,820 | Bản sao scene lịch sử; không phải stage mặc định. |
| scenes/fabric_feed_cutting_world.before_remove_table_perforations.usda | 922,145 | Bản sao scene lịch sử; không phải stage mặc định. |
| scenes/fabric_feed_cutting_world.before_ridge_fix.usda | 958,264 | Bản sao scene lịch sử; không phải stage mặc định. |
| scenes/fabric_feed_cutting_world.usda | 2,066,624 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scenes/felt_reference_sample.usdc | 9,839,838 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scenes/fixed_head_cutting_world.usda | 4,930,477 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scenes/he_thong_cat_ni.blend | 2,946,504 | Asset Blender nhị phân; chỉ kiểm kê metadata, không giải mã đầy đủ. |
| scenes/he_thong_cat_ni.blend1 | 2,938,839 | Asset Blender nhị phân; chỉ kiểm kê metadata, không giải mã đầy đủ. |
| scenes/jetson_light_world.usda | 2,816 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scenes/physx_felt_only_test.usda | 1,913,809 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scenes/physx_felt_test.usda | 1,945,340 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scenes/world.usda | 5,816 | USD scene/cache/asset; xem Chương 5 và Phụ lục B. |
| scripts/02_create_prim_tree.py | 1,773 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/03_create_basic_geometry.py | 5,682 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/__pycache__/02_create_prim_tree.cpython-314.pyc | 3,464 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/__pycache__/03_create_basic_geometry.cpython-314.pyc | 8,152 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/__pycache__/open_stage.cpython-314.pyc | 4,774 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/container_init.sh | 1,877 | Launcher hoặc entrypoint shell. |
| scripts/cutting/01_create_cutting_scene.py | 31,224 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/02_validate_cutting_scene.py | 13,247 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/03_play_cutting_demo.py | 634 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/05_create_fixed_head_scene.py | 18,166 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/06_validate_fixed_head_scene.py | 5,181 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/07_fixed_head_controls.py | 3,540 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/08_render_fixed_head.py | 5,161 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/09_create_fabric_feed_scene.py | 16,436 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/10_validate_fabric_feed_scene.py | 5,669 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/11_fabric_feed_controls.py | 4,794 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/12_render_fabric_feed.py | 4,994 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/13_convert_to_two_stepper_wheels.py | 7,686 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/14_validate_two_stepper_wheels.py | 2,995 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/15_upgrade_physical_cutting.py | 23,145 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/16_validate_physical_cutting.py | 9,099 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/17_rebuild_compliant_felt.py | 42,031 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/18_render_compliant_felt.py | 3,570 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/19_check_shell.py | 699 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/20_validate_compliant_felt.py | 33,736 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/21_sync_cloth_cache_clock.py | 541 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/22_finalize_candidate.py | 1,718 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/24_fill_large_table_openings.py | 4,430 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/25_compact_direct_wheel_mount.py | 11,503 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/27_render_pinch_diagnostic.py | 2,097 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/28_scrub_candidate_animation.py | 1,382 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/29_add_pinch_contact_skins.py | 1,219 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/30_apply_pending_mechanism_layout.py | 1,497 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/30_physx_felt_setup.py | 34,985 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/31_physx_felt_smoke_test.py | 4,569 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/cutting/__pycache__/01_create_cutting_scene.cpython-311.pyc | 48,867 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/01_create_cutting_scene.cpython-314.pyc | 48,009 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/02_validate_cutting_scene.cpython-314.pyc | 18,827 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/03_play_cutting_demo.cpython-314.pyc | 1,756 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/04_capture_line_camera.cpython-314.pyc | 4,553 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/05_create_fixed_head_scene.cpython-314.pyc | 33,757 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/06_validate_fixed_head_scene.cpython-314.pyc | 10,494 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/07_fixed_head_controls.cpython-314.pyc | 7,044 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/08_render_fixed_head.cpython-314.pyc | 9,490 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/09_create_fabric_feed_scene.cpython-314.pyc | 31,522 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/10_validate_fabric_feed_scene.cpython-314.pyc | 10,795 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/11_fabric_feed_controls.cpython-314.pyc | 9,876 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/12_render_fabric_feed.cpython-314.pyc | 9,894 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/13_convert_to_two_stepper_wheels.cpython-314.pyc | 13,429 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/14_validate_two_stepper_wheels.cpython-314.pyc | 5,868 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/15_upgrade_physical_cutting.cpython-314.pyc | 44,684 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/16_validate_physical_cutting.cpython-314.pyc | 16,769 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/17_rebuild_compliant_felt.cpython-311.pyc | 75,003 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/17_rebuild_compliant_felt.cpython-314.pyc | 71,661 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/18_render_compliant_felt.cpython-314.pyc | 7,175 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/19_check_shell.cpython-314.pyc | 2,000 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/20_validate_compliant_felt.cpython-314.pyc | 53,119 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/21_sync_cloth_cache_clock.cpython-314.pyc | 1,327 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/22_finalize_candidate.cpython-314.pyc | 2,606 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/23_remove_table_perforations.cpython-314.pyc | 4,034 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/24_fill_large_table_openings.cpython-314.pyc | 8,597 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/25_compact_direct_wheel_mount.cpython-314.pyc | 19,087 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/27_render_pinch_diagnostic.cpython-314.pyc | 4,192 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/28_scrub_candidate_animation.cpython-314.pyc | 2,163 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/29_add_pinch_contact_skins.cpython-314.pyc | 1,960 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/30_apply_pending_mechanism_layout.cpython-314.pyc | 2,398 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/30_physx_felt_setup.cpython-314.pyc | 60,494 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/31_physx_felt_smoke_test.cpython-314.pyc | 8,476 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/__pycache__/test_path_planner.cpython-314.pyc | 4,744 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/cutting/test_path_planner.py | 1,438 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/jetson_light/01_create_structure.py | 2,552 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/jetson_light/02_create_basic_geometry.py | 7,693 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/jetson_light/03_test_scene.py | 2,739 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/jetson_light/04_test_light.py | 2,841 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/jetson_light/05_virtual_light_sensor.py | 1,946 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/jetson_light/06_automatic_light_control.py | 2,812 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/jetson_light/06_daylight_sensor_simulation.py | 9,496 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/jetson_light/06_live_automatic_light_control.py | 4,343 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/jetson_light/07_isaac_ros2_light_bridge.py | 3,871 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/jetson_light/08_jetson_light_controller.py | 2,639 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| scripts/jetson_light/__pycache__/01_create_structure.cpython-314.pyc | 3,151 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/jetson_light/__pycache__/02_create_basic_geometry.cpython-314.pyc | 10,937 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/jetson_light/__pycache__/03_test_scene.cpython-314.pyc | 3,709 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/jetson_light/__pycache__/04_test_light.cpython-314.pyc | 4,332 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/jetson_light/__pycache__/05_virtual_light_sensor.cpython-314.pyc | 3,385 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/jetson_light/__pycache__/06_automatic_light_control.cpython-314.pyc | 4,834 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/jetson_light/__pycache__/06_daylight_sensor_simulation.cpython-314.pyc | 14,034 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/jetson_light/__pycache__/06_live_automatic_light_control.cpython-314.pyc | 6,393 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/jetson_light/__pycache__/07_isaac_ros2_light_bridge.cpython-314.pyc | 8,110 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/jetson_light/__pycache__/08_jetson_light_controller.cpython-314.pyc | 5,734 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| scripts/open_stage.py | 3,271 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| src/.gitkeep | 1 | File hỗ trợ hoặc cấu hình. |
| src/cutting/__init__.py | 196 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| src/cutting/__pycache__/__init__.cpython-311.pyc | 393 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| src/cutting/__pycache__/__init__.cpython-314.pyc | 360 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| src/cutting/__pycache__/fabric_feed_planner.cpython-311.pyc | 10,617 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| src/cutting/__pycache__/fabric_feed_planner.cpython-314.pyc | 10,578 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| src/cutting/__pycache__/felt_shell.cpython-311.pyc | 45,199 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| src/cutting/__pycache__/felt_shell.cpython-314.pyc | 45,333 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| src/cutting/__pycache__/fixed_head_planner.cpython-311.pyc | 26,488 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| src/cutting/__pycache__/fixed_head_planner.cpython-314.pyc | 26,156 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| src/cutting/__pycache__/path_planner.cpython-311.pyc | 16,187 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| src/cutting/__pycache__/path_planner.cpython-314.pyc | 17,739 | Bytecode/cache sinh tự động; không dùng làm nguồn sự thật khi còn file .py. |
| src/cutting/fabric_feed_planner.py | 6,181 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| src/cutting/felt_shell.py | 27,105 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| src/cutting/fixed_head_planner.py | 13,356 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| src/cutting/path_planner.py | 10,415 | Mã Python; hồ sơ chi tiết trong Chương 4 và Phụ lục C. |
| start_isaac.sh | 4,377 | Launcher hoặc entrypoint shell. |

# Phụ lục B Danh sách Prim

Danh sách hợp nhất dưới đây giữ mọi Prim path duy nhất trong các USDA đọc được. Cột scene count cho biết số file USDA có path đó. Các cache USDC là binary crate; Prim /Cloth, /Cloth/FinishedPart, /Cloth/Offcut được xác nhận từ code tạo và reference, không từ parser text.

| Prim path | Type | Số scene | Ví dụ file dòng |
| --- | --- | --- | --- |
| /World | Xform | 21 | isaac_projects/scenes/jetson_light_world.usda:7; scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:38; +19 file |
| /World/AirTable | Xform | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2507; scenes/fabric_feed_before_compliant_promotion.usda:2478; +13 file |
| /World/AirTable/Ambient | DomeLight | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6658; scenes/fabric_feed_before_straightening_rollers.usda:26035; +10 file |
| /World/AirTable/Bridge | Cube | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2530; scenes/fabric_feed_before_compliant_promotion.usda:2527; +13 file |
| /World/AirTable/FixedWheelFrameY2 | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2568; scenes/fabric_feed_before_straightening_rollers.usda:2568; +10 file |
| /World/AirTable/FrontWheelLeft | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2792; scenes/fabric_feed_before_straightening_rollers.usda:2646; +10 file |
| /World/AirTable/FrontWheelLeft/Coupling | Cylinder | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2901; scenes/fabric_feed_before_straightening_rollers.usda:5232; +10 file |
| /World/AirTable/FrontWheelLeft/Fork | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2857; scenes/fabric_feed_before_straightening_rollers.usda:5188; +10 file |
| /World/AirTable/FrontWheelLeft/ForkArmA | Cube | 9 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2868; scenes/fabric_feed_before_straightening_rollers.usda:5199; +7 file |
| /World/AirTable/FrontWheelLeft/ForkArmB | Cube | 9 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2879; scenes/fabric_feed_before_straightening_rollers.usda:5210; +7 file |
| /World/AirTable/FrontWheelLeft/Rotor | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2808; scenes/fabric_feed_before_straightening_rollers.usda:2662; +10 file |
| /World/AirTable/FrontWheelLeft/Rotor/Hub | Cylinder | 4 | scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda:5003; scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:2801; +2 file |
| /World/AirTable/FrontWheelLeft/Rotor/Hub | Mesh | 8 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2829; scenes/fabric_feed_before_straightening_rollers.usda:5160; +6 file |
| /World/AirTable/FrontWheelLeft/Rotor/IndexMark | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2845; scenes/fabric_feed_before_straightening_rollers.usda:5176; +10 file |
| /World/AirTable/FrontWheelLeft/Rotor/Rubber | Cylinder | 4 | scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda:4990; scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:2788; +2 file |
| /World/AirTable/FrontWheelLeft/Rotor/Rubber | Mesh | 8 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2813; scenes/fabric_feed_before_straightening_rollers.usda:5144; +6 file |
| /World/AirTable/FrontWheelLeft/StepperMotor | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2890; scenes/fabric_feed_before_straightening_rollers.usda:5221; +10 file |
| /World/AirTable/FrontWheelRight | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3213; scenes/fabric_feed_before_straightening_rollers.usda:7875; +10 file |
| /World/AirTable/FrontWheelRight/Coupling | Cylinder | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3322; scenes/fabric_feed_before_straightening_rollers.usda:10461; +10 file |
| /World/AirTable/FrontWheelRight/Fork | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3278; scenes/fabric_feed_before_straightening_rollers.usda:10417; +10 file |
| /World/AirTable/FrontWheelRight/ForkArmA | Cube | 9 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3289; scenes/fabric_feed_before_straightening_rollers.usda:10428; +7 file |
| /World/AirTable/FrontWheelRight/ForkArmB | Cube | 9 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3300; scenes/fabric_feed_before_straightening_rollers.usda:10439; +7 file |
| /World/AirTable/FrontWheelRight/Rotor | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3229; scenes/fabric_feed_before_straightening_rollers.usda:7891; +10 file |
| /World/AirTable/FrontWheelRight/Rotor/Hub | Cylinder | 4 | scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda:9897; scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:7695; +2 file |
| /World/AirTable/FrontWheelRight/Rotor/Hub | Mesh | 8 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3250; scenes/fabric_feed_before_straightening_rollers.usda:10389; +6 file |
| /World/AirTable/FrontWheelRight/Rotor/IndexMark | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3266; scenes/fabric_feed_before_straightening_rollers.usda:10405; +10 file |
| /World/AirTable/FrontWheelRight/Rotor/Rubber | Cylinder | 4 | scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda:9884; scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:7682; +2 file |
| /World/AirTable/FrontWheelRight/Rotor/Rubber | Mesh | 8 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3234; scenes/fabric_feed_before_straightening_rollers.usda:10373; +6 file |
| /World/AirTable/FrontWheelRight/StepperMotor | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3311; scenes/fabric_feed_before_straightening_rollers.usda:10450; +10 file |
| /World/AirTable/HeadDrop | Cube | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2542; scenes/fabric_feed_before_compliant_promotion.usda:2539; +13 file |
| /World/AirTable/HeadPlate | Cube | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2553; scenes/fabric_feed_before_compliant_promotion.usda:2550; +13 file |
| /World/AirTable/HoldRollerFrame | Xform | 1 | scenes/fabric_feed_before_straightening_rollers.usda:13039 |
| /World/AirTable/HoldRollerFrame/CrossBar | Cube | 1 | scenes/fabric_feed_before_straightening_rollers.usda:13046 |
| /World/AirTable/HoldRollerFrame/FrameFixedJoint | PhysicsFixedJoint | 1 | scenes/fabric_feed_before_straightening_rollers.usda:13079 |
| /World/AirTable/HoldRollerFrame/LinkA | Cube | 1 | scenes/fabric_feed_before_straightening_rollers.usda:13057 |
| /World/AirTable/HoldRollerFrame/LinkB | Cube | 1 | scenes/fabric_feed_before_straightening_rollers.usda:13068 |
| /World/AirTable/HoldRollerLeft | Xform | 1 | scenes/fabric_feed_before_straightening_rollers.usda:13122 |
| /World/AirTable/HoldRollerLeft/Fork | Cube | 1 | scenes/fabric_feed_before_straightening_rollers.usda:18129 |
| /World/AirTable/HoldRollerLeft/ForkArmA | Cube | 1 | scenes/fabric_feed_before_straightening_rollers.usda:18140 |
| /World/AirTable/HoldRollerLeft/ForkArmB | Cube | 1 | scenes/fabric_feed_before_straightening_rollers.usda:18151 |
| /World/AirTable/HoldRollerLeft/Rotor | Xform | 1 | scenes/fabric_feed_before_straightening_rollers.usda:15613 |
| /World/AirTable/HoldRollerLeft/Rotor/Hub | Mesh | 1 | scenes/fabric_feed_before_straightening_rollers.usda:18112 |
| /World/AirTable/HoldRollerLeft/Rotor/Rubber | Mesh | 1 | scenes/fabric_feed_before_straightening_rollers.usda:18096 |
| /World/AirTable/HoldRollerRight | Xform | 1 | scenes/fabric_feed_before_straightening_rollers.usda:18199 |
| /World/AirTable/HoldRollerRight/Fork | Cube | 1 | scenes/fabric_feed_before_straightening_rollers.usda:23206 |
| /World/AirTable/HoldRollerRight/ForkArmA | Cube | 1 | scenes/fabric_feed_before_straightening_rollers.usda:23217 |
| /World/AirTable/HoldRollerRight/ForkArmB | Cube | 1 | scenes/fabric_feed_before_straightening_rollers.usda:23228 |
| /World/AirTable/HoldRollerRight/Rotor | Xform | 1 | scenes/fabric_feed_before_straightening_rollers.usda:20690 |
| /World/AirTable/HoldRollerRight/Rotor/Hub | Mesh | 1 | scenes/fabric_feed_before_straightening_rollers.usda:23189 |
| /World/AirTable/HoldRollerRight/Rotor/Rubber | Mesh | 1 | scenes/fabric_feed_before_straightening_rollers.usda:23173 |
| /World/AirTable/HoldSuspensionLeft | Xform | 1 | scenes/fabric_feed_before_straightening_rollers.usda:13086 |
| /World/AirTable/HoldSuspensionLeft/CoilSpring | BasisCurves | 1 | scenes/fabric_feed_before_straightening_rollers.usda:13107 |
| /World/AirTable/HoldSuspensionLeft/GuideShaft | Cylinder | 1 | scenes/fabric_feed_before_straightening_rollers.usda:13094 |
| /World/AirTable/HoldSuspensionRight | Xform | 1 | scenes/fabric_feed_before_straightening_rollers.usda:18163 |
| /World/AirTable/HoldSuspensionRight/CoilSpring | BasisCurves | 1 | scenes/fabric_feed_before_straightening_rollers.usda:18184 |
| /World/AirTable/HoldSuspensionRight/GuideShaft | Cylinder | 1 | scenes/fabric_feed_before_straightening_rollers.usda:18171 |
| /World/AirTable/InspectionCamera | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6460; scenes/fabric_feed_before_straightening_rollers.usda:25837; +10 file |
| /World/AirTable/InspectionCamera/Housing | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6465; scenes/fabric_feed_before_straightening_rollers.usda:25842; +10 file |
| /World/AirTable/InspectionCamera/Lens | Cylinder | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6477; scenes/fabric_feed_before_straightening_rollers.usda:25854; +10 file |
| /World/AirTable/InspectionCamera/Sensor | Camera | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6503; scenes/fabric_feed_before_straightening_rollers.usda:25880; +10 file |
| /World/AirTable/InspectionCamera/Support | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6491; scenes/fabric_feed_before_straightening_rollers.usda:25868; +10 file |
| /World/AirTable/LED1 | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6534; scenes/fabric_feed_before_straightening_rollers.usda:25911; +10 file |
| /World/AirTable/LED1/Diffuser | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6553; scenes/fabric_feed_before_straightening_rollers.usda:25930; +10 file |
| /World/AirTable/LED1/Housing | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6541; scenes/fabric_feed_before_straightening_rollers.usda:25918; +10 file |
| /World/AirTable/LED1/Light | RectLight | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6577; scenes/fabric_feed_before_straightening_rollers.usda:25954; +10 file |
| /World/AirTable/LED1/Support | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6565; scenes/fabric_feed_before_straightening_rollers.usda:25942; +10 file |
| /World/AirTable/LED2 | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6589; scenes/fabric_feed_before_straightening_rollers.usda:25966; +10 file |
| /World/AirTable/LED2/Diffuser | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6606; scenes/fabric_feed_before_straightening_rollers.usda:25983; +10 file |
| /World/AirTable/LED2/Housing | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6594; scenes/fabric_feed_before_straightening_rollers.usda:25971; +10 file |
| /World/AirTable/LED2/Light | RectLight | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6630; scenes/fabric_feed_before_straightening_rollers.usda:26007; +10 file |
| /World/AirTable/LED2/Support | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6618; scenes/fabric_feed_before_straightening_rollers.usda:25995; +10 file |
| /World/AirTable/LowerPinchRollerLeft | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2915; scenes/fabric_feed_before_straightening_rollers.usda:5246; +10 file |
| /World/AirTable/LowerPinchRollerLeft/BearingA | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2968; scenes/fabric_feed_before_straightening_rollers.usda:7776; +10 file |
| /World/AirTable/LowerPinchRollerLeft/BearingB | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2979; scenes/fabric_feed_before_straightening_rollers.usda:7787; +10 file |
| /World/AirTable/LowerPinchRollerLeft/Rotor | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2930; scenes/fabric_feed_before_straightening_rollers.usda:5261; +10 file |
| /World/AirTable/LowerPinchRollerLeft/Rotor/Hub | Cylinder | 4 | scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda:7417; scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:5215; +2 file |
| /World/AirTable/LowerPinchRollerLeft/Rotor/Hub | Mesh | 8 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2951; scenes/fabric_feed_before_straightening_rollers.usda:7759; +6 file |
| /World/AirTable/LowerPinchRollerLeft/Rotor/IndexMark | Cube | 5 | scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda:7430; scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:5228; +3 file |
| /World/AirTable/LowerPinchRollerLeft/Rotor/Rubber | Cylinder | 4 | scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda:7404; scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:5202; +2 file |
| /World/AirTable/LowerPinchRollerLeft/Rotor/Rubber | Mesh | 8 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2935; scenes/fabric_feed_before_straightening_rollers.usda:7743; +6 file |
| /World/AirTable/LowerPinchRollerLeft/TableMount | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2990; scenes/fabric_feed_before_straightening_rollers.usda:7798; +10 file |
| /World/AirTable/LowerPinchRollerRight | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3336; scenes/fabric_feed_before_straightening_rollers.usda:10475; +10 file |
| /World/AirTable/LowerPinchRollerRight/BearingA | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3389; scenes/fabric_feed_before_straightening_rollers.usda:13005; +10 file |
| /World/AirTable/LowerPinchRollerRight/BearingB | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3400; scenes/fabric_feed_before_straightening_rollers.usda:13016; +10 file |
| /World/AirTable/LowerPinchRollerRight/Rotor | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3351; scenes/fabric_feed_before_straightening_rollers.usda:10490; +10 file |
| /World/AirTable/LowerPinchRollerRight/Rotor/Hub | Cylinder | 4 | scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda:12311; scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:10109; +2 file |
| /World/AirTable/LowerPinchRollerRight/Rotor/Hub | Mesh | 8 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3372; scenes/fabric_feed_before_straightening_rollers.usda:12988; +6 file |
| /World/AirTable/LowerPinchRollerRight/Rotor/IndexMark | Cube | 5 | scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda:12324; scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:10122; +3 file |
| /World/AirTable/LowerPinchRollerRight/Rotor/Rubber | Cylinder | 4 | scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda:12298; scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:10096; +2 file |
| /World/AirTable/LowerPinchRollerRight/Rotor/Rubber | Mesh | 8 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3356; scenes/fabric_feed_before_straightening_rollers.usda:12972; +6 file |
| /World/AirTable/LowerPinchRollerRight/TableMount | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3411; scenes/fabric_feed_before_straightening_rollers.usda:13027; +10 file |
| /World/AirTable/OffcutChute | Cube | 2 | scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:10180; scenes/fabric_feed_cutting_world.before_remove_table_perforations.usda:12380 |
| /World/AirTable/OffcutSplitter | Cube | 2 | scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:10168; scenes/fabric_feed_cutting_world.before_remove_table_perforations.usda:12368 |
| /World/AirTable/Overhead | RectLight | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6664; scenes/fabric_feed_before_straightening_rollers.usda:26041; +10 file |
| /World/AirTable/PinchAccessSlotLeft | Cube | 2 | scenes/fabric_feed_before_compliant_promotion.usda:2486; scenes/fabric_feed_before_compliant_v2.usda:2486 |
| /World/AirTable/PinchAccessSlotRight | Cube | 2 | scenes/fabric_feed_before_compliant_promotion.usda:2499; scenes/fabric_feed_before_compliant_v2.usda:2499 |
| /World/AirTable/PinchContactSkinLeft | Mesh | 1 | scenes/fabric_feed_cutting_world.before_hold_rollers.usda:13048 |
| /World/AirTable/PinchContactSkinRight | Mesh | 1 | scenes/fabric_feed_cutting_world.before_hold_rollers.usda:13066 |
| /World/AirTable/Post0 | Cube | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2518; scenes/fabric_feed_before_compliant_promotion.usda:2515; +13 file |
| /World/AirTable/SensorRailY | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6522; scenes/fabric_feed_before_straightening_rollers.usda:25899; +10 file |
| /World/AirTable/SensorRailY_01 | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6643; scenes/fabric_feed_before_straightening_rollers.usda:26020; +10 file |
| /World/AirTable/StraighteningRollerLeft | Xform | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3423; scenes/fabric_feed_compliant_candidate.usda:13331; +4 file |
| /World/AirTable/StraighteningRollerLeft/Axle | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3511; scenes/fabric_feed_compliant_candidate.usda:20838; +4 file |
| /World/AirTable/StraighteningRollerLeft/AxleCapA | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3524; scenes/fabric_feed_compliant_candidate.usda:20851; +4 file |
| /World/AirTable/StraighteningRollerLeft/AxleCapB | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3537; scenes/fabric_feed_compliant_candidate.usda:20864; +4 file |
| /World/AirTable/StraighteningRollerLeft/FabricGuideBar | BasisCurves | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3614; scenes/fabric_feed_compliant_candidate.usda:20941; +4 file |
| /World/AirTable/StraighteningRollerLeft/Fork | Cube | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3550; scenes/fabric_feed_compliant_candidate.usda:20877; +4 file |
| /World/AirTable/StraighteningRollerLeft/ForkArmA | Cube | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3561; scenes/fabric_feed_compliant_candidate.usda:20888; +4 file |
| /World/AirTable/StraighteningRollerLeft/ForkArmB | Cube | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3572; scenes/fabric_feed_compliant_candidate.usda:20899; +4 file |
| /World/AirTable/StraighteningRollerLeft/Rotor | Xform | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3455; scenes/fabric_feed_compliant_candidate.usda:18312; +4 file |
| /World/AirTable/StraighteningRollerLeft/Rotor/Hub | Mesh | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3483; scenes/fabric_feed_compliant_candidate.usda:20810; +4 file |
| /World/AirTable/StraighteningRollerLeft/Rotor/IndexMark | Cube | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3499; scenes/fabric_feed_compliant_candidate.usda:20826; +4 file |
| /World/AirTable/StraighteningRollerLeft/Rotor/Rubber | Mesh | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3467; scenes/fabric_feed_compliant_candidate.usda:20794; +4 file |
| /World/AirTable/StraighteningRollerLeft/SwivelHead | Xform | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3583; scenes/fabric_feed_compliant_candidate.usda:20910; +4 file |
| /World/AirTable/StraighteningRollerLeft/SwivelHead/InnerRace | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3587; scenes/fabric_feed_compliant_candidate.usda:20914; +4 file |
| /World/AirTable/StraighteningRollerLeft/SwivelHead/Kingpin | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3600; scenes/fabric_feed_compliant_candidate.usda:20927; +4 file |
| /World/AirTable/StraighteningRollerRight | Xform | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3643; scenes/fabric_feed_compliant_candidate.usda:20966; +4 file |
| /World/AirTable/StraighteningRollerRight/Axle | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3731; scenes/fabric_feed_compliant_candidate.usda:28473; +4 file |
| /World/AirTable/StraighteningRollerRight/AxleCapA | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3744; scenes/fabric_feed_compliant_candidate.usda:28486; +4 file |
| /World/AirTable/StraighteningRollerRight/AxleCapB | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3757; scenes/fabric_feed_compliant_candidate.usda:28499; +4 file |
| /World/AirTable/StraighteningRollerRight/FabricGuideBar | BasisCurves | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3834; scenes/fabric_feed_compliant_candidate.usda:28576; +4 file |
| /World/AirTable/StraighteningRollerRight/Fork | Cube | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3770; scenes/fabric_feed_compliant_candidate.usda:28512; +4 file |
| /World/AirTable/StraighteningRollerRight/ForkArmA | Cube | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3781; scenes/fabric_feed_compliant_candidate.usda:28523; +4 file |
| /World/AirTable/StraighteningRollerRight/ForkArmB | Cube | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3792; scenes/fabric_feed_compliant_candidate.usda:28534; +4 file |
| /World/AirTable/StraighteningRollerRight/Metal | Material | 1 | scenes/physx_felt_test.usda:4315 |
| /World/AirTable/StraighteningRollerRight/Rotor | Xform | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3675; scenes/fabric_feed_compliant_candidate.usda:25947; +4 file |
| /World/AirTable/StraighteningRollerRight/Rotor/Hub | Mesh | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3703; scenes/fabric_feed_compliant_candidate.usda:28445; +4 file |
| /World/AirTable/StraighteningRollerRight/Rotor/IndexMark | Cube | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3719; scenes/fabric_feed_compliant_candidate.usda:28461; +4 file |
| /World/AirTable/StraighteningRollerRight/Rotor/Rubber | Mesh | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3687; scenes/fabric_feed_compliant_candidate.usda:28429; +4 file |
| /World/AirTable/StraighteningRollerRight/Rubber | Material | 1 | scenes/physx_felt_test.usda:4327 |
| /World/AirTable/StraighteningRollerRight/SwivelHead | Xform | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3803; scenes/fabric_feed_compliant_candidate.usda:28545; +4 file |
| /World/AirTable/StraighteningRollerRight/SwivelHead/InnerRace | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3807; scenes/fabric_feed_compliant_candidate.usda:28549; +4 file |
| /World/AirTable/StraighteningRollerRight/SwivelHead/Kingpin | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3820; scenes/fabric_feed_compliant_candidate.usda:28562; +4 file |
| /World/AirTable/SuspensionLeft | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2581; scenes/fabric_feed_before_straightening_rollers.usda:2581; +10 file |
| /World/AirTable/SuspensionLeft/CoilSpring | BasisCurves | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2616; scenes/fabric_feed_before_straightening_rollers.usda:2615; +10 file |
| /World/AirTable/SuspensionLeft/FrameFixedJoint | PhysicsFixedJoint | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2630; scenes/fabric_feed_before_straightening_rollers.usda:2629; +10 file |
| /World/AirTable/SuspensionLeft/GuideShaft | Cylinder | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2590; scenes/fabric_feed_before_straightening_rollers.usda:2589; +10 file |
| /World/AirTable/SuspensionLeft/OffsetBracket | Cube | 3 | scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda:2589; scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:387; +1 file |
| /World/AirTable/SuspensionLeft/StraightenerMount | Xform | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2646; scenes/fabric_feed_compliant_candidate.usda:2646; +4 file |
| /World/AirTable/SuspensionLeft/StraightenerMount/DirectLink | Xform | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2648; scenes/fabric_feed_compliant_candidate.usda:2648; +4 file |
| /World/AirTable/SuspensionLeft/StraightenerMount/DirectLink/Beam | Cube | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2653; scenes/fabric_feed_compliant_candidate.usda:2653; +4 file |
| /World/AirTable/SuspensionLeft/StraightenerMount/DirectLink/FrontEye | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2690; scenes/fabric_feed_compliant_candidate.usda:2690; +4 file |
| /World/AirTable/SuspensionLeft/StraightenerMount/DirectLink/FrontEyePin | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2703; scenes/fabric_feed_compliant_candidate.usda:2703; +4 file |
| /World/AirTable/SuspensionLeft/StraightenerMount/DirectLink/RearEye | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2664; scenes/fabric_feed_compliant_candidate.usda:2664; +4 file |
| /World/AirTable/SuspensionLeft/StraightenerMount/DirectLink/RearEyePin | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2677; scenes/fabric_feed_compliant_candidate.usda:2677; +4 file |
| /World/AirTable/SuspensionLeft/StraightenerMount/StraightenerSteeringJoint | PhysicsRevoluteJoint | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2771; scenes/fabric_feed_compliant_candidate.usda:2771; +4 file |
| /World/AirTable/SuspensionLeft/StraightenerMount/VerticalPost | Xform | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2717; scenes/fabric_feed_compliant_candidate.usda:2717; +4 file |
| /World/AirTable/SuspensionLeft/StraightenerMount/VerticalPost/BearingCap | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2746; scenes/fabric_feed_compliant_candidate.usda:2746; +4 file |
| /World/AirTable/SuspensionLeft/StraightenerMount/VerticalPost/CornerGusset | Mesh | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2759; scenes/fabric_feed_compliant_candidate.usda:2759; +4 file |
| /World/AirTable/SuspensionLeft/StraightenerMount/VerticalPost/SteeringHousing | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2733; scenes/fabric_feed_compliant_candidate.usda:2733; +4 file |
| /World/AirTable/SuspensionLeft/StraightenerMount/VerticalPost/Stem | Cube | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2722; scenes/fabric_feed_compliant_candidate.usda:2722; +4 file |
| /World/AirTable/SuspensionLeft/UpperSpringSeat | Cylinder | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2603; scenes/fabric_feed_before_straightening_rollers.usda:2602; +10 file |
| /World/AirTable/SuspensionLeft/WheelFixedJoint | PhysicsFixedJoint | 9 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2638; scenes/fabric_feed_before_straightening_rollers.usda:2637; +7 file |
| /World/AirTable/SuspensionRight | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3002; scenes/fabric_feed_before_straightening_rollers.usda:7810; +10 file |
| /World/AirTable/SuspensionRight/CoilSpring | BasisCurves | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3037; scenes/fabric_feed_before_straightening_rollers.usda:7844; +10 file |
| /World/AirTable/SuspensionRight/FrameFixedJoint | PhysicsFixedJoint | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3051; scenes/fabric_feed_before_straightening_rollers.usda:7858; +10 file |
| /World/AirTable/SuspensionRight/GuideShaft | Cylinder | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3011; scenes/fabric_feed_before_straightening_rollers.usda:7818; +10 file |
| /World/AirTable/SuspensionRight/OffsetBracket | Cube | 3 | scenes/fabric_feed_cutting_world.before_compact_direct_mount.usda:7483; scenes/fabric_feed_cutting_world.before_fill_large_openings.usda:5281; +1 file |
| /World/AirTable/SuspensionRight/StraightenerMount | Xform | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3067; scenes/fabric_feed_compliant_candidate.usda:8021; +4 file |
| /World/AirTable/SuspensionRight/StraightenerMount/DirectLink | Xform | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3069; scenes/fabric_feed_compliant_candidate.usda:8023; +4 file |
| /World/AirTable/SuspensionRight/StraightenerMount/DirectLink/Beam | Cube | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3074; scenes/fabric_feed_compliant_candidate.usda:8028; +4 file |
| /World/AirTable/SuspensionRight/StraightenerMount/DirectLink/FrontEye | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3111; scenes/fabric_feed_compliant_candidate.usda:8065; +4 file |
| /World/AirTable/SuspensionRight/StraightenerMount/DirectLink/FrontEyePin | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3124; scenes/fabric_feed_compliant_candidate.usda:8078; +4 file |
| /World/AirTable/SuspensionRight/StraightenerMount/DirectLink/RearEye | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3085; scenes/fabric_feed_compliant_candidate.usda:8039; +4 file |
| /World/AirTable/SuspensionRight/StraightenerMount/DirectLink/RearEyePin | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3098; scenes/fabric_feed_compliant_candidate.usda:8052; +4 file |
| /World/AirTable/SuspensionRight/StraightenerMount/StraightenerSteeringJoint | PhysicsRevoluteJoint | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3192; scenes/fabric_feed_compliant_candidate.usda:8146; +4 file |
| /World/AirTable/SuspensionRight/StraightenerMount/VerticalPost | Xform | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3138; scenes/fabric_feed_compliant_candidate.usda:8092; +4 file |
| /World/AirTable/SuspensionRight/StraightenerMount/VerticalPost/BearingCap | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3167; scenes/fabric_feed_compliant_candidate.usda:8121; +4 file |
| /World/AirTable/SuspensionRight/StraightenerMount/VerticalPost/CornerGusset | Mesh | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3180; scenes/fabric_feed_compliant_candidate.usda:8134; +4 file |
| /World/AirTable/SuspensionRight/StraightenerMount/VerticalPost/SteeringHousing | Cylinder | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3154; scenes/fabric_feed_compliant_candidate.usda:8108; +4 file |
| /World/AirTable/SuspensionRight/StraightenerMount/VerticalPost/Stem | Cube | 6 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3143; scenes/fabric_feed_compliant_candidate.usda:8097; +4 file |
| /World/AirTable/SuspensionRight/UpperSpringSeat | Cylinder | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3024; scenes/fabric_feed_before_straightening_rollers.usda:7831; +10 file |
| /World/AirTable/SuspensionRight/WheelFixedJoint | PhysicsFixedJoint | 9 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3059; scenes/fabric_feed_before_straightening_rollers.usda:7866; +7 file |
| /World/AirTable/ZSlide | Xform | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:3866; scenes/fabric_feed_before_straightening_rollers.usda:23243; +10 file |
| /World/AirTable/ZSlide/Blade | Mesh | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6429; scenes/fabric_feed_before_straightening_rollers.usda:25806; +10 file |
| /World/AirTable/ZSlide/Body | Cylinder | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6391; scenes/fabric_feed_before_straightening_rollers.usda:25768; +10 file |
| /World/AirTable/ZSlide/Carriage | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6379; scenes/fabric_feed_before_straightening_rollers.usda:25756; +10 file |
| /World/AirTable/ZSlide/Chuck | Cylinder | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6416; scenes/fabric_feed_before_straightening_rollers.usda:25793; +10 file |
| /World/AirTable/ZSlide/Guide1 | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6355; scenes/fabric_feed_before_straightening_rollers.usda:25732; +10 file |
| /World/AirTable/ZSlide/Guide2 | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6367; scenes/fabric_feed_before_straightening_rollers.usda:25744; +10 file |
| /World/AirTable/ZSlide/Motor200W | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6404; scenes/fabric_feed_before_straightening_rollers.usda:25781; +10 file |
| /World/AirTable/ZSlide/SensorRailY_02 | Cube | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6444; scenes/fabric_feed_before_straightening_rollers.usda:25821; +10 file |
| /World/CameraBracket | Cube | 1 | scenes/fixed_head_cutting_world.usda:5074 |
| /World/CameraBracketCrossTie | Cube | 1 | scenes/fixed_head_cutting_world.usda:5085 |
| /World/Carrier | Xform | 1 | scenes/fixed_head_cutting_world.usda:5234 |
| /World/Carrier/Ambient | DomeLight | 1 | scenes/fixed_head_cutting_world.usda:66840 |
| /World/Carrier/Battery | Cube | 1 | scenes/fixed_head_cutting_world.usda:13960 |
| /World/Carrier/Deck | Cube | 1 | scenes/fixed_head_cutting_world.usda:13905 |
| /World/Carrier/FrameX0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:13916 |
| /World/Carrier/FrameX1 | Cube | 1 | scenes/fixed_head_cutting_world.usda:13927 |
| /World/Carrier/FrameY0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:13938 |
| /World/Carrier/FrameY1 | Cube | 1 | scenes/fixed_head_cutting_world.usda:13949 |
| /World/Carrier/NestSupport0 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:13995 |
| /World/Carrier/NestSupport1 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14008 |
| /World/Carrier/NestSupport2 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14021 |
| /World/Carrier/Overhead | RectLight | 1 | scenes/fixed_head_cutting_world.usda:66846 |
| /World/Carrier/SacrificialMat | Mesh | 1 | scenes/fixed_head_cutting_world.usda:14047 |
| /World/Carrier/SacrificialMat/Fastener0 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14244 |
| /World/Carrier/SacrificialMat/Fastener1 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14270 |
| /World/Carrier/SacrificialMat/Fastener2 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14296 |
| /World/Carrier/SacrificialMat/FeltBlank | Mesh | 1 | scenes/fixed_head_cutting_world.usda:14062 |
| /World/Carrier/SacrificialMat/Module1 | Xform | 1 | scenes/fixed_head_cutting_world.usda:14312 |
| /World/Carrier/SacrificialMat/Module1/DeckBracket | Cube | 1 | scenes/fixed_head_cutting_world.usda:14317 |
| /World/Carrier/SacrificialMat/Module1/SteerBearing | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14339 |
| /World/Carrier/SacrificialMat/Module1/SteerServo | Cube | 1 | scenes/fixed_head_cutting_world.usda:14328 |
| /World/Carrier/SacrificialMat/Module1/Steering | Xform | 1 | scenes/fixed_head_cutting_world.usda:14352 |
| /World/Carrier/SacrificialMat/Module1/Steering/DriveAxle | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:23043 |
| /World/Carrier/SacrificialMat/Module1/Steering/DriveMotor | Cube | 1 | scenes/fixed_head_cutting_world.usda:23056 |
| /World/Carrier/SacrificialMat/Module1/Steering/Fork0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:23021 |
| /World/Carrier/SacrificialMat/Module1/Steering/Fork2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:23032 |
| /World/Carrier/SacrificialMat/Module1/Steering/Rotor | Xform | 1 | scenes/fixed_head_cutting_world.usda:23067 |
| /World/Carrier/SacrificialMat/Module1/Steering/Rotor/Hub | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:27416 |
| /World/Carrier/SacrificialMat/Module1/Steering/Rotor/IndexMark | Cube | 1 | scenes/fixed_head_cutting_world.usda:27429 |
| /World/Carrier/SacrificialMat/Module1/Steering/Rotor/Tire | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:27403 |
| /World/Carrier/SacrificialMat/Module2 | Xform | 1 | scenes/fixed_head_cutting_world.usda:27443 |
| /World/Carrier/SacrificialMat/Module2/DeckBracket | Cube | 1 | scenes/fixed_head_cutting_world.usda:27448 |
| /World/Carrier/SacrificialMat/Module2/SteerBearing | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:27470 |
| /World/Carrier/SacrificialMat/Module2/SteerServo | Cube | 1 | scenes/fixed_head_cutting_world.usda:27459 |
| /World/Carrier/SacrificialMat/Module2/Steering | Xform | 1 | scenes/fixed_head_cutting_world.usda:27483 |
| /World/Carrier/SacrificialMat/Module2/Steering/DriveAxle | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:36174 |
| /World/Carrier/SacrificialMat/Module2/Steering/DriveMotor | Cube | 1 | scenes/fixed_head_cutting_world.usda:36187 |
| /World/Carrier/SacrificialMat/Module2/Steering/Fork0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:36152 |
| /World/Carrier/SacrificialMat/Module2/Steering/Fork2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:36163 |
| /World/Carrier/SacrificialMat/Module2/Steering/Rotor | Xform | 1 | scenes/fixed_head_cutting_world.usda:36198 |
| /World/Carrier/SacrificialMat/Module2/Steering/Rotor/Hub | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:40547 |
| /World/Carrier/SacrificialMat/Module2/Steering/Rotor/IndexMark | Cube | 1 | scenes/fixed_head_cutting_world.usda:40560 |
| /World/Carrier/SacrificialMat/Module2/Steering/Rotor/Tire | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:40534 |
| /World/Carrier/SacrificialMat/Module3 | Xform | 1 | scenes/fixed_head_cutting_world.usda:40574 |
| /World/Carrier/SacrificialMat/Module3/DeckBracket | Cube | 1 | scenes/fixed_head_cutting_world.usda:40579 |
| /World/Carrier/SacrificialMat/Module3/SteerBearing | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:40601 |
| /World/Carrier/SacrificialMat/Module3/SteerServo | Cube | 1 | scenes/fixed_head_cutting_world.usda:40590 |
| /World/Carrier/SacrificialMat/Module3/Steering | Xform | 1 | scenes/fixed_head_cutting_world.usda:40614 |
| /World/Carrier/SacrificialMat/Module3/Steering/DriveAxle | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:49305 |
| /World/Carrier/SacrificialMat/Module3/Steering/DriveMotor | Cube | 1 | scenes/fixed_head_cutting_world.usda:49318 |
| /World/Carrier/SacrificialMat/Module3/Steering/Fork0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:49283 |
| /World/Carrier/SacrificialMat/Module3/Steering/Fork2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:49294 |
| /World/Carrier/SacrificialMat/Module3/Steering/Rotor | Xform | 1 | scenes/fixed_head_cutting_world.usda:49329 |
| /World/Carrier/SacrificialMat/Module3/Steering/Rotor/Hub | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:53678 |
| /World/Carrier/SacrificialMat/Module3/Steering/Rotor/IndexMark | Cube | 1 | scenes/fixed_head_cutting_world.usda:53691 |
| /World/Carrier/SacrificialMat/Module3/Steering/Rotor/Tire | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:53665 |
| /World/Carrier/SacrificialMat/Module4 | Xform | 1 | scenes/fixed_head_cutting_world.usda:53705 |
| /World/Carrier/SacrificialMat/Module4/DeckBracket | Cube | 1 | scenes/fixed_head_cutting_world.usda:53710 |
| /World/Carrier/SacrificialMat/Module4/SteerBearing | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:53732 |
| /World/Carrier/SacrificialMat/Module4/SteerServo | Cube | 1 | scenes/fixed_head_cutting_world.usda:53721 |
| /World/Carrier/SacrificialMat/Module4/Steering | Xform | 1 | scenes/fixed_head_cutting_world.usda:53745 |
| /World/Carrier/SacrificialMat/Module4/Steering/DriveAxle | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:62436 |
| /World/Carrier/SacrificialMat/Module4/Steering/DriveMotor | Cube | 1 | scenes/fixed_head_cutting_world.usda:62449 |
| /World/Carrier/SacrificialMat/Module4/Steering/Fork0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:62414 |
| /World/Carrier/SacrificialMat/Module4/Steering/Fork2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:62425 |
| /World/Carrier/SacrificialMat/Module4/Steering/Rotor | Xform | 1 | scenes/fixed_head_cutting_world.usda:62460 |
| /World/Carrier/SacrificialMat/Module4/Steering/Rotor/Hub | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:66809 |
| /World/Carrier/SacrificialMat/Module4/Steering/Rotor/IndexMark | Cube | 1 | scenes/fixed_head_cutting_world.usda:66822 |
| /World/Carrier/SacrificialMat/Module4/Steering/Rotor/Tire | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:66796 |
| /World/Carrier/SacrificialMat/Pad0 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14231 |
| /World/Carrier/SacrificialMat/Pad1 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14257 |
| /World/Carrier/SacrificialMat/Pad2 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14283 |
| /World/Carrier/SacrificialMat/RidgeInner | Mesh | 1 | scenes/fixed_head_cutting_world.usda:14073 |
| /World/Carrier/SacrificialMat/RidgeOuter | Mesh | 1 | scenes/fixed_head_cutting_world.usda:14084 |
| /World/Carrier/SacrificialMat/RidgeOuter/H0 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14097 |
| /World/Carrier/SacrificialMat/RidgeOuter/H1 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14110 |
| /World/Carrier/SacrificialMat/RidgeOuter/H2 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14123 |
| /World/Carrier/SacrificialMat/RidgeOuter/H3 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14136 |
| /World/Carrier/SacrificialMat/RidgeOuter/H4 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14149 |
| /World/Carrier/SacrificialMat/RidgeOuter/H5 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14162 |
| /World/Carrier/SacrificialMat/RidgeOuter/H6 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14175 |
| /World/Carrier/SacrificialMat/RidgeOuter/H7 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14188 |
| /World/Carrier/SacrificialMat/RidgeOuter/H8 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14201 |
| /World/Carrier/SacrificialMat/RidgeOuter/H9 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:14214 |
| /World/Carrier/ShapedNest | Mesh | 1 | scenes/fixed_head_cutting_world.usda:14034 |
| /World/Carrier/VacuumPump | Cube | 1 | scenes/fixed_head_cutting_world.usda:13971 |
| /World/Carrier/VacuumReservoir | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:13982 |
| /World/Controller | Xform | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:6675; scenes/fabric_feed_before_compliant_promotion.usda:82332; +14 file |
| /World/Controller/omni_kit_widget_viewport_ViewportTexture_0 | RenderProduct | 4 | scenes/fabric_feed_before_compliant_promotion.usda:95360; scenes/fabric_feed_before_compliant_v2.usda:95360; +2 file |
| /World/CuttingMat | Material | 1 | scenes/fixed_head_cutting_world.usda:147 |
| /World/CuttingMat/Shader | Shader | 1 | scenes/fixed_head_cutting_world.usda:151 |
| /World/CuttingSystem | Xform | 2 | scenes/cutting_world.usda:4414; scenes/cutting_world_user_edit_backup.usda:2488 |
| /World/CuttingSystem/Dome | DomeLight | 2 | scenes/cutting_world.usda:4430; scenes/cutting_world_user_edit_backup.usda:2504 |
| /World/CuttingSystem/InspectionLight | RectLight | 2 | scenes/cutting_world.usda:4436; scenes/cutting_world_user_edit_backup.usda:2510 |
| /World/CuttingSystem/InspectionLight/omni_kit_widget_viewport_ViewportTexture_0 | RenderProduct | 1 | scenes/cutting_world_user_edit_backup.usda:2536 |
| /World/Dark | Material | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:117; scenes/fabric_feed_before_compliant_promotion.usda:117; +14 file |
| /World/Dark/Shader | Shader | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:121; scenes/fabric_feed_before_compliant_promotion.usda:121; +14 file |
| /World/Felt | Material | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:162; scenes/fabric_feed_before_compliant_promotion.usda:162; +14 file |
| /World/Felt/Shader | Shader | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:166; scenes/fabric_feed_before_compliant_promotion.usda:166; +14 file |
| /World/FixedBridge | Cube | 1 | scenes/fixed_head_cutting_world.usda:368 |
| /World/Fixture | Material | 1 | scenes/fixed_head_cutting_world.usda:132 |
| /World/Fixture/Shader | Shader | 1 | scenes/fixed_head_cutting_world.usda:136 |
| /World/Floor | Cube | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:227; scenes/fabric_feed_before_compliant_promotion.usda:196; +14 file |
| /World/Floor | Material | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:42; scenes/fabric_feed_before_compliant_promotion.usda:42; +14 file |
| /World/Floor/Shader | Shader | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:46; scenes/fabric_feed_before_compliant_promotion.usda:46; +14 file |
| /World/Foot0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:335 |
| /World/Foot2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:357 |
| /World/InspectionCamera | Xform | 1 | scenes/fixed_head_cutting_world.usda:5022 |
| /World/InspectionCamera/Housing | Cube | 1 | scenes/fixed_head_cutting_world.usda:5027 |
| /World/InspectionCamera/Lens | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:5038 |
| /World/InspectionCamera/Sensor | Camera | 1 | scenes/fixed_head_cutting_world.usda:5051 |
| /World/InspectionLED0 | Xform | 1 | scenes/fixed_head_cutting_world.usda:5096 |
| /World/InspectionLED0/Diffuser | Cube | 1 | scenes/fixed_head_cutting_world.usda:5113 |
| /World/InspectionLED0/Housing | Cube | 1 | scenes/fixed_head_cutting_world.usda:5101 |
| /World/InspectionLED0/Light | RectLight | 1 | scenes/fixed_head_cutting_world.usda:5125 |
| /World/InspectionLED0/Support | Cube | 1 | scenes/fixed_head_cutting_world.usda:5136 |
| /World/InspectionLED0/TopBracket | Cube | 1 | scenes/fixed_head_cutting_world.usda:5147 |
| /World/InspectionLED2 | Xform | 1 | scenes/fixed_head_cutting_world.usda:5159 |
| /World/InspectionLED2/Diffuser | Cube | 1 | scenes/fixed_head_cutting_world.usda:5176 |
| /World/InspectionLED2/Housing | Cube | 1 | scenes/fixed_head_cutting_world.usda:5164 |
| /World/InspectionLED2/Light | RectLight | 1 | scenes/fixed_head_cutting_world.usda:5188 |
| /World/InspectionLED2/Support | Cube | 1 | scenes/fixed_head_cutting_world.usda:5199 |
| /World/InspectionLED2/TopBracket | Cube | 1 | scenes/fixed_head_cutting_world.usda:5210 |
| /World/LEDBracketCrossTie | Cube | 1 | scenes/fixed_head_cutting_world.usda:5222 |
| /World/LEDDiffuser | Material | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:177; scenes/fabric_feed_before_compliant_promotion.usda:177; +14 file |
| /World/LEDDiffuser/Shader | Shader | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:181; scenes/fabric_feed_before_compliant_promotion.usda:181; +14 file |
| /World/Lamp | Xform | 3 | isaac_projects/scenes/jetson_light_world.usda:57; scenes/jetson_light_world.usda:57; +1 file |
| /World/Lamp/LampBody | Sphere | 3 | isaac_projects/scenes/jetson_light_world.usda:59; scenes/jetson_light_world.usda:59; +1 file |
| /World/Lamp/Light | SphereLight | 3 | isaac_projects/scenes/jetson_light_world.usda:68; scenes/jetson_light_world.usda:68; +1 file |
| /World/Leg1 | Cube | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:250; scenes/fabric_feed_before_compliant_promotion.usda:233; +13 file |
| /World/Leg2 | Cube | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:261; scenes/fabric_feed_before_compliant_promotion.usda:244; +13 file |
| /World/Leg3 | Cube | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:272; scenes/fabric_feed_before_compliant_promotion.usda:255; +13 file |
| /World/Leg4 | Cube | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:283; scenes/fabric_feed_before_compliant_promotion.usda:266; +13 file |
| /World/Leg4/P_0_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:296; scenes/fabric_feed_before_compliant_promotion.usda:279; +12 file |
| /World/Leg4/P_0_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:309; scenes/fabric_feed_before_compliant_promotion.usda:292; +12 file |
| /World/Leg4/P_0_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:426; scenes/fabric_feed_before_compliant_promotion.usda:409; +12 file |
| /World/Leg4/P_0_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:439; scenes/fabric_feed_before_compliant_promotion.usda:422; +12 file |
| /World/Leg4/P_0_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:452; scenes/fabric_feed_before_compliant_promotion.usda:435; +12 file |
| /World/Leg4/P_0_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:322; scenes/fabric_feed_before_compliant_promotion.usda:305; +12 file |
| /World/Leg4/P_0_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:335; scenes/fabric_feed_before_compliant_promotion.usda:318; +12 file |
| /World/Leg4/P_0_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:348; scenes/fabric_feed_before_compliant_promotion.usda:331; +12 file |
| /World/Leg4/P_0_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:361; scenes/fabric_feed_before_compliant_promotion.usda:344; +12 file |
| /World/Leg4/P_0_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:374; scenes/fabric_feed_before_compliant_promotion.usda:357; +12 file |
| /World/Leg4/P_0_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:387; scenes/fabric_feed_before_compliant_promotion.usda:370; +12 file |
| /World/Leg4/P_0_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:400; scenes/fabric_feed_before_compliant_promotion.usda:383; +12 file |
| /World/Leg4/P_0_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:413; scenes/fabric_feed_before_compliant_promotion.usda:396; +12 file |
| /World/Leg4/P_10_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1986; scenes/fabric_feed_before_compliant_promotion.usda:1969; +12 file |
| /World/Leg4/P_10_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1999; scenes/fabric_feed_before_compliant_promotion.usda:1982; +12 file |
| /World/Leg4/P_10_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2116; scenes/fabric_feed_before_compliant_promotion.usda:2099; +12 file |
| /World/Leg4/P_10_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2129; scenes/fabric_feed_before_compliant_promotion.usda:2112; +12 file |
| /World/Leg4/P_10_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2142; scenes/fabric_feed_before_compliant_promotion.usda:2125; +12 file |
| /World/Leg4/P_10_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2012; scenes/fabric_feed_before_compliant_promotion.usda:1995; +12 file |
| /World/Leg4/P_10_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2025; scenes/fabric_feed_before_compliant_promotion.usda:2008; +12 file |
| /World/Leg4/P_10_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2038; scenes/fabric_feed_before_compliant_promotion.usda:2021; +12 file |
| /World/Leg4/P_10_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2051; scenes/fabric_feed_before_compliant_promotion.usda:2034; +12 file |
| /World/Leg4/P_10_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2064; scenes/fabric_feed_before_compliant_promotion.usda:2047; +12 file |
| /World/Leg4/P_10_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2077; scenes/fabric_feed_before_compliant_promotion.usda:2060; +12 file |
| /World/Leg4/P_10_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2090; scenes/fabric_feed_before_compliant_promotion.usda:2073; +12 file |
| /World/Leg4/P_10_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2103; scenes/fabric_feed_before_compliant_promotion.usda:2086; +12 file |
| /World/Leg4/P_11_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2155; scenes/fabric_feed_before_compliant_promotion.usda:2138; +12 file |
| /World/Leg4/P_11_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2168; scenes/fabric_feed_before_compliant_promotion.usda:2151; +12 file |
| /World/Leg4/P_11_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2285; scenes/fabric_feed_before_compliant_promotion.usda:2268; +12 file |
| /World/Leg4/P_11_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2298; scenes/fabric_feed_before_compliant_promotion.usda:2281; +12 file |
| /World/Leg4/P_11_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2311; scenes/fabric_feed_before_compliant_promotion.usda:2294; +12 file |
| /World/Leg4/P_11_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2181; scenes/fabric_feed_before_compliant_promotion.usda:2164; +12 file |
| /World/Leg4/P_11_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2194; scenes/fabric_feed_before_compliant_promotion.usda:2177; +12 file |
| /World/Leg4/P_11_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2207; scenes/fabric_feed_before_compliant_promotion.usda:2190; +12 file |
| /World/Leg4/P_11_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2220; scenes/fabric_feed_before_compliant_promotion.usda:2203; +12 file |
| /World/Leg4/P_11_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2233; scenes/fabric_feed_before_compliant_promotion.usda:2216; +12 file |
| /World/Leg4/P_11_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2246; scenes/fabric_feed_before_compliant_promotion.usda:2229; +12 file |
| /World/Leg4/P_11_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2259; scenes/fabric_feed_before_compliant_promotion.usda:2242; +12 file |
| /World/Leg4/P_11_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2272; scenes/fabric_feed_before_compliant_promotion.usda:2255; +12 file |
| /World/Leg4/P_12_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2324; scenes/fabric_feed_before_compliant_promotion.usda:2307; +12 file |
| /World/Leg4/P_12_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2337; scenes/fabric_feed_before_compliant_promotion.usda:2320; +12 file |
| /World/Leg4/P_12_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2454; scenes/fabric_feed_before_compliant_promotion.usda:2437; +12 file |
| /World/Leg4/P_12_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2467; scenes/fabric_feed_before_compliant_promotion.usda:2450; +12 file |
| /World/Leg4/P_12_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2480; scenes/fabric_feed_before_compliant_promotion.usda:2463; +12 file |
| /World/Leg4/P_12_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2350; scenes/fabric_feed_before_compliant_promotion.usda:2333; +12 file |
| /World/Leg4/P_12_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2363; scenes/fabric_feed_before_compliant_promotion.usda:2346; +12 file |
| /World/Leg4/P_12_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2376; scenes/fabric_feed_before_compliant_promotion.usda:2359; +12 file |
| /World/Leg4/P_12_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2389; scenes/fabric_feed_before_compliant_promotion.usda:2372; +12 file |
| /World/Leg4/P_12_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2402; scenes/fabric_feed_before_compliant_promotion.usda:2385; +12 file |
| /World/Leg4/P_12_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2415; scenes/fabric_feed_before_compliant_promotion.usda:2398; +12 file |
| /World/Leg4/P_12_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2428; scenes/fabric_feed_before_compliant_promotion.usda:2411; +12 file |
| /World/Leg4/P_12_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2441; scenes/fabric_feed_before_compliant_promotion.usda:2424; +12 file |
| /World/Leg4/P_1_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:465; scenes/fabric_feed_before_compliant_promotion.usda:448; +12 file |
| /World/Leg4/P_1_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:478; scenes/fabric_feed_before_compliant_promotion.usda:461; +12 file |
| /World/Leg4/P_1_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:595; scenes/fabric_feed_before_compliant_promotion.usda:578; +12 file |
| /World/Leg4/P_1_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:608; scenes/fabric_feed_before_compliant_promotion.usda:591; +12 file |
| /World/Leg4/P_1_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:621; scenes/fabric_feed_before_compliant_promotion.usda:604; +12 file |
| /World/Leg4/P_1_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:491; scenes/fabric_feed_before_compliant_promotion.usda:474; +12 file |
| /World/Leg4/P_1_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:504; scenes/fabric_feed_before_compliant_promotion.usda:487; +12 file |
| /World/Leg4/P_1_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:517; scenes/fabric_feed_before_compliant_promotion.usda:500; +12 file |
| /World/Leg4/P_1_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:530; scenes/fabric_feed_before_compliant_promotion.usda:513; +12 file |
| /World/Leg4/P_1_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:543; scenes/fabric_feed_before_compliant_promotion.usda:526; +12 file |
| /World/Leg4/P_1_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:556; scenes/fabric_feed_before_compliant_promotion.usda:539; +12 file |
| /World/Leg4/P_1_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:569; scenes/fabric_feed_before_compliant_promotion.usda:552; +12 file |
| /World/Leg4/P_1_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:582; scenes/fabric_feed_before_compliant_promotion.usda:565; +12 file |
| /World/Leg4/P_2_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:634; scenes/fabric_feed_before_compliant_promotion.usda:617; +12 file |
| /World/Leg4/P_2_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:647; scenes/fabric_feed_before_compliant_promotion.usda:630; +12 file |
| /World/Leg4/P_2_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:764; scenes/fabric_feed_before_compliant_promotion.usda:747; +12 file |
| /World/Leg4/P_2_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:777; scenes/fabric_feed_before_compliant_promotion.usda:760; +12 file |
| /World/Leg4/P_2_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:790; scenes/fabric_feed_before_compliant_promotion.usda:773; +12 file |
| /World/Leg4/P_2_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:660; scenes/fabric_feed_before_compliant_promotion.usda:643; +12 file |
| /World/Leg4/P_2_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:673; scenes/fabric_feed_before_compliant_promotion.usda:656; +12 file |
| /World/Leg4/P_2_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:686; scenes/fabric_feed_before_compliant_promotion.usda:669; +12 file |
| /World/Leg4/P_2_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:699; scenes/fabric_feed_before_compliant_promotion.usda:682; +12 file |
| /World/Leg4/P_2_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:712; scenes/fabric_feed_before_compliant_promotion.usda:695; +12 file |
| /World/Leg4/P_2_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:725; scenes/fabric_feed_before_compliant_promotion.usda:708; +12 file |
| /World/Leg4/P_2_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:738; scenes/fabric_feed_before_compliant_promotion.usda:721; +12 file |
| /World/Leg4/P_2_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:751; scenes/fabric_feed_before_compliant_promotion.usda:734; +12 file |
| /World/Leg4/P_3_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:803; scenes/fabric_feed_before_compliant_promotion.usda:786; +12 file |
| /World/Leg4/P_3_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:816; scenes/fabric_feed_before_compliant_promotion.usda:799; +12 file |
| /World/Leg4/P_3_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:933; scenes/fabric_feed_before_compliant_promotion.usda:916; +12 file |
| /World/Leg4/P_3_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:946; scenes/fabric_feed_before_compliant_promotion.usda:929; +12 file |
| /World/Leg4/P_3_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:959; scenes/fabric_feed_before_compliant_promotion.usda:942; +12 file |
| /World/Leg4/P_3_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:829; scenes/fabric_feed_before_compliant_promotion.usda:812; +12 file |
| /World/Leg4/P_3_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:842; scenes/fabric_feed_before_compliant_promotion.usda:825; +12 file |
| /World/Leg4/P_3_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:855; scenes/fabric_feed_before_compliant_promotion.usda:838; +12 file |
| /World/Leg4/P_3_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:868; scenes/fabric_feed_before_compliant_promotion.usda:851; +12 file |
| /World/Leg4/P_3_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:881; scenes/fabric_feed_before_compliant_promotion.usda:864; +12 file |
| /World/Leg4/P_3_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:894; scenes/fabric_feed_before_compliant_promotion.usda:877; +12 file |
| /World/Leg4/P_3_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:907; scenes/fabric_feed_before_compliant_promotion.usda:890; +12 file |
| /World/Leg4/P_3_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:920; scenes/fabric_feed_before_compliant_promotion.usda:903; +12 file |
| /World/Leg4/P_4_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:972; scenes/fabric_feed_before_compliant_promotion.usda:955; +12 file |
| /World/Leg4/P_4_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:985; scenes/fabric_feed_before_compliant_promotion.usda:968; +12 file |
| /World/Leg4/P_4_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1102; scenes/fabric_feed_before_compliant_promotion.usda:1085; +12 file |
| /World/Leg4/P_4_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1115; scenes/fabric_feed_before_compliant_promotion.usda:1098; +12 file |
| /World/Leg4/P_4_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1128; scenes/fabric_feed_before_compliant_promotion.usda:1111; +12 file |
| /World/Leg4/P_4_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:998; scenes/fabric_feed_before_compliant_promotion.usda:981; +12 file |
| /World/Leg4/P_4_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1011; scenes/fabric_feed_before_compliant_promotion.usda:994; +12 file |
| /World/Leg4/P_4_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1024; scenes/fabric_feed_before_compliant_promotion.usda:1007; +12 file |
| /World/Leg4/P_4_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1037; scenes/fabric_feed_before_compliant_promotion.usda:1020; +12 file |
| /World/Leg4/P_4_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1050; scenes/fabric_feed_before_compliant_promotion.usda:1033; +12 file |
| /World/Leg4/P_4_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1063; scenes/fabric_feed_before_compliant_promotion.usda:1046; +12 file |
| /World/Leg4/P_4_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1076; scenes/fabric_feed_before_compliant_promotion.usda:1059; +12 file |
| /World/Leg4/P_4_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1089; scenes/fabric_feed_before_compliant_promotion.usda:1072; +12 file |
| /World/Leg4/P_5_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1141; scenes/fabric_feed_before_compliant_promotion.usda:1124; +12 file |
| /World/Leg4/P_5_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1154; scenes/fabric_feed_before_compliant_promotion.usda:1137; +12 file |
| /World/Leg4/P_5_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1271; scenes/fabric_feed_before_compliant_promotion.usda:1254; +12 file |
| /World/Leg4/P_5_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1284; scenes/fabric_feed_before_compliant_promotion.usda:1267; +12 file |
| /World/Leg4/P_5_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1297; scenes/fabric_feed_before_compliant_promotion.usda:1280; +12 file |
| /World/Leg4/P_5_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1167; scenes/fabric_feed_before_compliant_promotion.usda:1150; +12 file |
| /World/Leg4/P_5_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1180; scenes/fabric_feed_before_compliant_promotion.usda:1163; +12 file |
| /World/Leg4/P_5_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1193; scenes/fabric_feed_before_compliant_promotion.usda:1176; +12 file |
| /World/Leg4/P_5_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1206; scenes/fabric_feed_before_compliant_promotion.usda:1189; +12 file |
| /World/Leg4/P_5_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1219; scenes/fabric_feed_before_compliant_promotion.usda:1202; +12 file |
| /World/Leg4/P_5_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1232; scenes/fabric_feed_before_compliant_promotion.usda:1215; +12 file |
| /World/Leg4/P_5_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1245; scenes/fabric_feed_before_compliant_promotion.usda:1228; +12 file |
| /World/Leg4/P_5_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1258; scenes/fabric_feed_before_compliant_promotion.usda:1241; +12 file |
| /World/Leg4/P_6_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1310; scenes/fabric_feed_before_compliant_promotion.usda:1293; +12 file |
| /World/Leg4/P_6_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1323; scenes/fabric_feed_before_compliant_promotion.usda:1306; +12 file |
| /World/Leg4/P_6_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1440; scenes/fabric_feed_before_compliant_promotion.usda:1423; +12 file |
| /World/Leg4/P_6_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1453; scenes/fabric_feed_before_compliant_promotion.usda:1436; +12 file |
| /World/Leg4/P_6_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1466; scenes/fabric_feed_before_compliant_promotion.usda:1449; +12 file |
| /World/Leg4/P_6_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1336; scenes/fabric_feed_before_compliant_promotion.usda:1319; +12 file |
| /World/Leg4/P_6_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1349; scenes/fabric_feed_before_compliant_promotion.usda:1332; +12 file |
| /World/Leg4/P_6_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1362; scenes/fabric_feed_before_compliant_promotion.usda:1345; +12 file |
| /World/Leg4/P_6_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1375; scenes/fabric_feed_before_compliant_promotion.usda:1358; +12 file |
| /World/Leg4/P_6_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1388; scenes/fabric_feed_before_compliant_promotion.usda:1371; +12 file |
| /World/Leg4/P_6_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1401; scenes/fabric_feed_before_compliant_promotion.usda:1384; +12 file |
| /World/Leg4/P_6_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1414; scenes/fabric_feed_before_compliant_promotion.usda:1397; +12 file |
| /World/Leg4/P_6_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1427; scenes/fabric_feed_before_compliant_promotion.usda:1410; +12 file |
| /World/Leg4/P_7_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1479; scenes/fabric_feed_before_compliant_promotion.usda:1462; +12 file |
| /World/Leg4/P_7_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1492; scenes/fabric_feed_before_compliant_promotion.usda:1475; +12 file |
| /World/Leg4/P_7_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1609; scenes/fabric_feed_before_compliant_promotion.usda:1592; +12 file |
| /World/Leg4/P_7_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1622; scenes/fabric_feed_before_compliant_promotion.usda:1605; +12 file |
| /World/Leg4/P_7_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1635; scenes/fabric_feed_before_compliant_promotion.usda:1618; +12 file |
| /World/Leg4/P_7_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1505; scenes/fabric_feed_before_compliant_promotion.usda:1488; +12 file |
| /World/Leg4/P_7_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1518; scenes/fabric_feed_before_compliant_promotion.usda:1501; +12 file |
| /World/Leg4/P_7_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1531; scenes/fabric_feed_before_compliant_promotion.usda:1514; +12 file |
| /World/Leg4/P_7_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1544; scenes/fabric_feed_before_compliant_promotion.usda:1527; +12 file |
| /World/Leg4/P_7_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1557; scenes/fabric_feed_before_compliant_promotion.usda:1540; +12 file |
| /World/Leg4/P_7_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1570; scenes/fabric_feed_before_compliant_promotion.usda:1553; +12 file |
| /World/Leg4/P_7_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1583; scenes/fabric_feed_before_compliant_promotion.usda:1566; +12 file |
| /World/Leg4/P_7_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1596; scenes/fabric_feed_before_compliant_promotion.usda:1579; +12 file |
| /World/Leg4/P_8_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1648; scenes/fabric_feed_before_compliant_promotion.usda:1631; +12 file |
| /World/Leg4/P_8_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1661; scenes/fabric_feed_before_compliant_promotion.usda:1644; +12 file |
| /World/Leg4/P_8_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1778; scenes/fabric_feed_before_compliant_promotion.usda:1761; +12 file |
| /World/Leg4/P_8_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1791; scenes/fabric_feed_before_compliant_promotion.usda:1774; +12 file |
| /World/Leg4/P_8_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1804; scenes/fabric_feed_before_compliant_promotion.usda:1787; +12 file |
| /World/Leg4/P_8_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1674; scenes/fabric_feed_before_compliant_promotion.usda:1657; +12 file |
| /World/Leg4/P_8_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1687; scenes/fabric_feed_before_compliant_promotion.usda:1670; +12 file |
| /World/Leg4/P_8_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1700; scenes/fabric_feed_before_compliant_promotion.usda:1683; +12 file |
| /World/Leg4/P_8_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1713; scenes/fabric_feed_before_compliant_promotion.usda:1696; +12 file |
| /World/Leg4/P_8_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1726; scenes/fabric_feed_before_compliant_promotion.usda:1709; +12 file |
| /World/Leg4/P_8_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1739; scenes/fabric_feed_before_compliant_promotion.usda:1722; +12 file |
| /World/Leg4/P_8_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1752; scenes/fabric_feed_before_compliant_promotion.usda:1735; +12 file |
| /World/Leg4/P_8_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1765; scenes/fabric_feed_before_compliant_promotion.usda:1748; +12 file |
| /World/Leg4/P_9_0 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1817; scenes/fabric_feed_before_compliant_promotion.usda:1800; +12 file |
| /World/Leg4/P_9_1 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1830; scenes/fabric_feed_before_compliant_promotion.usda:1813; +12 file |
| /World/Leg4/P_9_10 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1947; scenes/fabric_feed_before_compliant_promotion.usda:1930; +12 file |
| /World/Leg4/P_9_11 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1960; scenes/fabric_feed_before_compliant_promotion.usda:1943; +12 file |
| /World/Leg4/P_9_12 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1973; scenes/fabric_feed_before_compliant_promotion.usda:1956; +12 file |
| /World/Leg4/P_9_2 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1843; scenes/fabric_feed_before_compliant_promotion.usda:1826; +12 file |
| /World/Leg4/P_9_3 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1856; scenes/fabric_feed_before_compliant_promotion.usda:1839; +12 file |
| /World/Leg4/P_9_4 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1869; scenes/fabric_feed_before_compliant_promotion.usda:1852; +12 file |
| /World/Leg4/P_9_5 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1882; scenes/fabric_feed_before_compliant_promotion.usda:1865; +12 file |
| /World/Leg4/P_9_6 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1895; scenes/fabric_feed_before_compliant_promotion.usda:1878; +12 file |
| /World/Leg4/P_9_7 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1908; scenes/fabric_feed_before_compliant_promotion.usda:1891; +12 file |
| /World/Leg4/P_9_8 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1921; scenes/fabric_feed_before_compliant_promotion.usda:1904; +12 file |
| /World/Leg4/P_9_9 | Cylinder | 14 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:1934; scenes/fabric_feed_before_compliant_promotion.usda:1917; +12 file |
| /World/Looks | Xform | 2 | scenes/cutting_world.usda:14; scenes/cutting_world_user_edit_backup.usda:40 |
| /World/Looks/Dark | Material | 2 | scenes/cutting_world.usda:106; scenes/cutting_world_user_edit_backup.usda:207 |
| /World/Looks/Dark/Shader | Shader | 2 | scenes/cutting_world.usda:110; scenes/cutting_world_user_edit_backup.usda:211 |
| /World/Looks/Felt | Material | 2 | scenes/cutting_world.usda:61; scenes/cutting_world_user_edit_backup.usda:87 |
| /World/Looks/Felt/Shader | Shader | 2 | scenes/cutting_world.usda:65; scenes/cutting_world_user_edit_backup.usda:91 |
| /World/Looks/Fixture | Material | 2 | scenes/cutting_world.usda:46; scenes/cutting_world_user_edit_backup.usda:72 |
| /World/Looks/Fixture/Shader | Shader | 2 | scenes/cutting_world.usda:50; scenes/cutting_world_user_edit_backup.usda:76 |
| /World/Looks/Floor | Material | 2 | scenes/cutting_world.usda:16; scenes/cutting_world_user_edit_backup.usda:42 |
| /World/Looks/Floor/Shader | Shader | 2 | scenes/cutting_world.usda:20; scenes/cutting_world_user_edit_backup.usda:46 |
| /World/Looks/MachineBlue | Material | 2 | scenes/cutting_world.usda:91; scenes/cutting_world_user_edit_backup.usda:192 |
| /World/Looks/MachineBlue/Shader | Shader | 2 | scenes/cutting_world.usda:95; scenes/cutting_world_user_edit_backup.usda:196 |
| /World/Looks/PathGreen | Material | 1 | scenes/cutting_world_user_edit_backup.usda:132 |
| /World/Looks/PathGreen/Shader | Shader | 1 | scenes/cutting_world_user_edit_backup.usda:136 |
| /World/Looks/PathRed | Material | 1 | scenes/cutting_world_user_edit_backup.usda:162 |
| /World/Looks/PathRed/Shader | Shader | 1 | scenes/cutting_world_user_edit_backup.usda:166 |
| /World/Looks/PathYellow | Material | 1 | scenes/cutting_world_user_edit_backup.usda:147 |
| /World/Looks/PathYellow/Shader | Shader | 1 | scenes/cutting_world_user_edit_backup.usda:151 |
| /World/Looks/RidgeInner | Material | 1 | scenes/cutting_world_user_edit_backup.usda:102 |
| /World/Looks/RidgeInner/Shader | Shader | 1 | scenes/cutting_world_user_edit_backup.usda:106 |
| /World/Looks/RidgeOuter | Material | 1 | scenes/cutting_world_user_edit_backup.usda:117 |
| /World/Looks/RidgeOuter/Shader | Shader | 1 | scenes/cutting_world_user_edit_backup.usda:121 |
| /World/Looks/SafetyOrange | Material | 2 | scenes/cutting_world.usda:121; scenes/cutting_world_user_edit_backup.usda:222 |
| /World/Looks/SafetyOrange/Shader | Shader | 2 | scenes/cutting_world.usda:125; scenes/cutting_world_user_edit_backup.usda:226 |
| /World/Looks/Steel | Material | 2 | scenes/cutting_world.usda:76; scenes/cutting_world_user_edit_backup.usda:177 |
| /World/Looks/Steel/Shader | Shader | 2 | scenes/cutting_world.usda:80; scenes/cutting_world_user_edit_backup.usda:181 |
| /World/Looks/Table | Material | 2 | scenes/cutting_world.usda:31; scenes/cutting_world_user_edit_backup.usda:57 |
| /World/Looks/Table/Shader | Shader | 2 | scenes/cutting_world.usda:35; scenes/cutting_world_user_edit_backup.usda:61 |
| /World/LooseFabric | Xform | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:16617; scenes/fabric_feed_before_compliant_promotion.usda:2563; +13 file |
| /World/LooseFabric/Ambient | DomeLight | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82315; scenes/fabric_feed_before_compliant_v2.usda:82315; +1 file |
| /World/LooseFabric/Blank | Cube | 1 | scenes/fabric_feed_cutting_world.before_physical_upgrade.usda:11208 |
| /World/LooseFabric/Blank | Mesh | 2 | scenes/fabric_feed_before_compliant_promotion.usda:11396; scenes/fabric_feed_before_compliant_v2.usda:11396 |
| /World/LooseFabric/EmbossedRidgeInner | Mesh | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11241; scenes/fabric_feed_before_compliant_v2.usda:11241; +1 file |
| /World/LooseFabric/EmbossedRidgeOuter | Mesh | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11252; scenes/fabric_feed_before_compliant_v2.usda:11252; +1 file |
| /World/LooseFabric/EmbossedRidgeOuter/H0 | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11265; scenes/fabric_feed_before_compliant_v2.usda:11265; +1 file |
| /World/LooseFabric/EmbossedRidgeOuter/H1 | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11278; scenes/fabric_feed_before_compliant_v2.usda:11278; +1 file |
| /World/LooseFabric/EmbossedRidgeOuter/H2 | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11291; scenes/fabric_feed_before_compliant_v2.usda:11291; +1 file |
| /World/LooseFabric/EmbossedRidgeOuter/H3 | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11304; scenes/fabric_feed_before_compliant_v2.usda:11304; +1 file |
| /World/LooseFabric/EmbossedRidgeOuter/H4 | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11317; scenes/fabric_feed_before_compliant_v2.usda:11317; +1 file |
| /World/LooseFabric/EmbossedRidgeOuter/H5 | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11330; scenes/fabric_feed_before_compliant_v2.usda:11330; +1 file |
| /World/LooseFabric/EmbossedRidgeOuter/H6 | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11343; scenes/fabric_feed_before_compliant_v2.usda:11343; +1 file |
| /World/LooseFabric/EmbossedRidgeOuter/H7 | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11356; scenes/fabric_feed_before_compliant_v2.usda:11356; +1 file |
| /World/LooseFabric/EmbossedRidgeOuter/H8 | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11369; scenes/fabric_feed_before_compliant_v2.usda:11369; +1 file |
| /World/LooseFabric/EmbossedRidgeOuter/H9 | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11382; scenes/fabric_feed_before_compliant_v2.usda:11382; +1 file |
| /World/LooseFabric/FinishedPart | Mesh | 2 | scenes/fabric_feed_before_compliant_promotion.usda:11412; scenes/fabric_feed_before_compliant_v2.usda:11412 |
| /World/LooseFabric/FixedWheelFrameY2 | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11449; scenes/fabric_feed_before_compliant_v2.usda:11449; +1 file |
| /World/LooseFabric/FrontWheelLeft | Xform | 3 | scenes/fabric_feed_before_compliant_promotion.usda:11461; scenes/fabric_feed_before_compliant_v2.usda:11461; +1 file |
| /World/LooseFabric/FrontWheelLeft/Fork | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:33177; scenes/fabric_feed_before_compliant_v2.usda:33177; +1 file |
| /World/LooseFabric/FrontWheelLeft/Rotor | Xform | 3 | scenes/fabric_feed_before_compliant_promotion.usda:28802; scenes/fabric_feed_before_compliant_v2.usda:28802; +1 file |
| /World/LooseFabric/FrontWheelLeft/Rotor/Hub | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:33150; scenes/fabric_feed_before_compliant_v2.usda:33150; +1 file |
| /World/LooseFabric/FrontWheelLeft/Rotor/HubCap | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:33163; scenes/fabric_feed_before_compliant_v2.usda:33163; +1 file |
| /World/LooseFabric/FrontWheelLeft/Rotor/RubberTire | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:33137; scenes/fabric_feed_before_compliant_v2.usda:33137; +1 file |
| /World/LooseFabric/FrontWheelLeft/SpringPlunger | Cylinder | 1 | scenes/fabric_feed_cutting_world.before_physical_upgrade.usda:24413 |
| /World/LooseFabric/FrontWheelLeft/StepperMotor | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:33188; scenes/fabric_feed_before_compliant_v2.usda:33188; +1 file |
| /World/LooseFabric/FrontWheelLeft/StepperShaft | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:33200; scenes/fabric_feed_before_compliant_v2.usda:33200; +1 file |
| /World/LooseFabric/FrontWheelRight | Xform | 3 | scenes/fabric_feed_before_compliant_promotion.usda:44564; scenes/fabric_feed_before_compliant_v2.usda:44564; +1 file |
| /World/LooseFabric/FrontWheelRight/Fork | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:66280; scenes/fabric_feed_before_compliant_v2.usda:66280; +1 file |
| /World/LooseFabric/FrontWheelRight/Rotor | Xform | 3 | scenes/fabric_feed_before_compliant_promotion.usda:61905; scenes/fabric_feed_before_compliant_v2.usda:61905; +1 file |
| /World/LooseFabric/FrontWheelRight/Rotor/Hub | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:66253; scenes/fabric_feed_before_compliant_v2.usda:66253; +1 file |
| /World/LooseFabric/FrontWheelRight/Rotor/HubCap | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:66266; scenes/fabric_feed_before_compliant_v2.usda:66266; +1 file |
| /World/LooseFabric/FrontWheelRight/Rotor/RubberTire | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:66240; scenes/fabric_feed_before_compliant_v2.usda:66240; +1 file |
| /World/LooseFabric/FrontWheelRight/SpringPlunger | Cylinder | 1 | scenes/fabric_feed_cutting_world.before_physical_upgrade.usda:41853 |
| /World/LooseFabric/FrontWheelRight/StepperMotor | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:66291; scenes/fabric_feed_before_compliant_v2.usda:66291; +1 file |
| /World/LooseFabric/FrontWheelRight/StepperShaft | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:66303; scenes/fabric_feed_before_compliant_v2.usda:66303; +1 file |
| /World/LooseFabric/InspectionCamera | Xform | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82117; scenes/fabric_feed_before_compliant_v2.usda:82117; +1 file |
| /World/LooseFabric/InspectionCamera/Housing | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82122; scenes/fabric_feed_before_compliant_v2.usda:82122; +1 file |
| /World/LooseFabric/InspectionCamera/Lens | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82134; scenes/fabric_feed_before_compliant_v2.usda:82134; +1 file |
| /World/LooseFabric/InspectionCamera/Sensor | Camera | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82160; scenes/fabric_feed_before_compliant_v2.usda:82160; +1 file |
| /World/LooseFabric/InspectionCamera/Support | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82148; scenes/fabric_feed_before_compliant_v2.usda:82148; +1 file |
| /World/LooseFabric/LED1 | Xform | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82191; scenes/fabric_feed_before_compliant_v2.usda:82191; +1 file |
| /World/LooseFabric/LED1/Diffuser | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82210; scenes/fabric_feed_before_compliant_v2.usda:82210; +1 file |
| /World/LooseFabric/LED1/Housing | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82198; scenes/fabric_feed_before_compliant_v2.usda:82198; +1 file |
| /World/LooseFabric/LED1/Light | RectLight | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82234; scenes/fabric_feed_before_compliant_v2.usda:82234; +1 file |
| /World/LooseFabric/LED1/Support | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82222; scenes/fabric_feed_before_compliant_v2.usda:82222; +1 file |
| /World/LooseFabric/LED2 | Xform | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82246; scenes/fabric_feed_before_compliant_v2.usda:82246; +1 file |
| /World/LooseFabric/LED2/Diffuser | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82263; scenes/fabric_feed_before_compliant_v2.usda:82263; +1 file |
| /World/LooseFabric/LED2/Housing | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82251; scenes/fabric_feed_before_compliant_v2.usda:82251; +1 file |
| /World/LooseFabric/LED2/Light | RectLight | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82287; scenes/fabric_feed_before_compliant_v2.usda:82287; +1 file |
| /World/LooseFabric/LED2/Support | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82275; scenes/fabric_feed_before_compliant_v2.usda:82275; +1 file |
| /World/LooseFabric/LowerPinchRollerLeft | Xform | 2 | scenes/fabric_feed_before_compliant_promotion.usda:33214; scenes/fabric_feed_before_compliant_v2.usda:33214 |
| /World/LooseFabric/LowerPinchRollerLeft/Fork | Cube | 2 | scenes/fabric_feed_before_compliant_promotion.usda:41916; scenes/fabric_feed_before_compliant_v2.usda:41916 |
| /World/LooseFabric/LowerPinchRollerLeft/Rotor | Xform | 2 | scenes/fabric_feed_before_compliant_promotion.usda:37554; scenes/fabric_feed_before_compliant_v2.usda:37554 |
| /World/LooseFabric/LowerPinchRollerLeft/Rotor/Hub | Cylinder | 2 | scenes/fabric_feed_before_compliant_promotion.usda:41902; scenes/fabric_feed_before_compliant_v2.usda:41902 |
| /World/LooseFabric/LowerPinchRollerLeft/Rotor/RubberRoller | Cylinder | 2 | scenes/fabric_feed_before_compliant_promotion.usda:41889; scenes/fabric_feed_before_compliant_v2.usda:41889 |
| /World/LooseFabric/LowerPinchRollerRight | Xform | 2 | scenes/fabric_feed_before_compliant_promotion.usda:66317; scenes/fabric_feed_before_compliant_v2.usda:66317 |
| /World/LooseFabric/LowerPinchRollerRight/Fork | Cube | 2 | scenes/fabric_feed_before_compliant_promotion.usda:75019; scenes/fabric_feed_before_compliant_v2.usda:75019 |
| /World/LooseFabric/LowerPinchRollerRight/Rotor | Xform | 2 | scenes/fabric_feed_before_compliant_promotion.usda:70657; scenes/fabric_feed_before_compliant_v2.usda:70657 |
| /World/LooseFabric/LowerPinchRollerRight/Rotor/Hub | Cylinder | 2 | scenes/fabric_feed_before_compliant_promotion.usda:75005; scenes/fabric_feed_before_compliant_v2.usda:75005 |
| /World/LooseFabric/LowerPinchRollerRight/Rotor/RubberRoller | Cylinder | 2 | scenes/fabric_feed_before_compliant_promotion.usda:74992; scenes/fabric_feed_before_compliant_v2.usda:74992 |
| /World/LooseFabric/Overhead | RectLight | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82321; scenes/fabric_feed_before_compliant_v2.usda:82321; +1 file |
| /World/LooseFabric/PhysxFeltTestSheet | Mesh | 2 | scenes/physx_felt_only_test.usda:16671; scenes/physx_felt_test.usda:17126 |
| /World/LooseFabric/PhysxFeltTestSheet/Felt | Material | 2 | scenes/physx_felt_only_test.usda:16745; scenes/physx_felt_test.usda:17200 |
| /World/LooseFabric/PhysxFeltTestSheet/FeltPBR | Material | 2 | scenes/physx_felt_only_test.usda:16697; scenes/physx_felt_test.usda:17152 |
| /World/LooseFabric/PhysxFeltTestSheet/FeltPBR/DiffuseTexture | Shader | 2 | scenes/physx_felt_only_test.usda:16721; scenes/physx_felt_test.usda:17176 |
| /World/LooseFabric/PhysxFeltTestSheet/FeltPBR/NormalTexture | Shader | 2 | scenes/physx_felt_only_test.usda:16730; scenes/physx_felt_test.usda:17185 |
| /World/LooseFabric/PhysxFeltTestSheet/FeltPBR/Shader | Shader | 2 | scenes/physx_felt_only_test.usda:16701; scenes/physx_felt_test.usda:17156 |
| /World/LooseFabric/PhysxFeltTestSheet/FeltPBR/UV | Shader | 2 | scenes/physx_felt_only_test.usda:16714; scenes/physx_felt_test.usda:17169 |
| /World/LooseFabric/SensorRailY | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82179; scenes/fabric_feed_before_compliant_v2.usda:82179; +1 file |
| /World/LooseFabric/SensorRailY_01 | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82300; scenes/fabric_feed_before_compliant_v2.usda:82300; +1 file |
| /World/LooseFabric/SeparatedScrap | Mesh | 2 | scenes/fabric_feed_before_compliant_promotion.usda:11429; scenes/fabric_feed_before_compliant_v2.usda:11429 |
| /World/LooseFabric/SuspensionLeft | Xform | 2 | scenes/fabric_feed_before_compliant_promotion.usda:41928; scenes/fabric_feed_before_compliant_v2.usda:41928 |
| /World/LooseFabric/SuspensionLeft/CoilSpring | BasisCurves | 2 | scenes/fabric_feed_before_compliant_promotion.usda:43681; scenes/fabric_feed_before_compliant_v2.usda:43681 |
| /World/LooseFabric/SuspensionLeft/GuideShaft | Cylinder | 2 | scenes/fabric_feed_before_compliant_promotion.usda:41933; scenes/fabric_feed_before_compliant_v2.usda:41933 |
| /World/LooseFabric/SuspensionRight | Xform | 2 | scenes/fabric_feed_before_compliant_promotion.usda:75031; scenes/fabric_feed_before_compliant_v2.usda:75031 |
| /World/LooseFabric/SuspensionRight/CoilSpring | BasisCurves | 2 | scenes/fabric_feed_before_compliant_promotion.usda:76784; scenes/fabric_feed_before_compliant_v2.usda:76784 |
| /World/LooseFabric/SuspensionRight/GuideShaft | Cylinder | 2 | scenes/fabric_feed_before_compliant_promotion.usda:75036; scenes/fabric_feed_before_compliant_v2.usda:75036 |
| /World/LooseFabric/ZSlide | Xform | 3 | scenes/fabric_feed_before_compliant_promotion.usda:77670; scenes/fabric_feed_before_compliant_v2.usda:77670; +1 file |
| /World/LooseFabric/ZSlide/Blade | Mesh | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82086; scenes/fabric_feed_before_compliant_v2.usda:82086; +1 file |
| /World/LooseFabric/ZSlide/Body | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82048; scenes/fabric_feed_before_compliant_v2.usda:82048; +1 file |
| /World/LooseFabric/ZSlide/Carriage | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82036; scenes/fabric_feed_before_compliant_v2.usda:82036; +1 file |
| /World/LooseFabric/ZSlide/Chuck | Cylinder | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82073; scenes/fabric_feed_before_compliant_v2.usda:82073; +1 file |
| /World/LooseFabric/ZSlide/Guide1 | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82012; scenes/fabric_feed_before_compliant_v2.usda:82012; +1 file |
| /World/LooseFabric/ZSlide/Guide2 | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82024; scenes/fabric_feed_before_compliant_v2.usda:82024; +1 file |
| /World/LooseFabric/ZSlide/Motor200W | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82061; scenes/fabric_feed_before_compliant_v2.usda:82061; +1 file |
| /World/LooseFabric/ZSlide/SensorRailY_02 | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:82101; scenes/fabric_feed_before_compliant_v2.usda:82101; +1 file |
| /World/LooseFabric/omni_kit_widget_viewport_ViewportTexture_0 | RenderProduct | 10 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:16663; scenes/fabric_feed_before_straightening_rollers.usda:36039; +8 file |
| /World/Machine | Xform | 2 | scenes/cutting_world.usda:368; scenes/cutting_world_user_edit_backup.usda:506 |
| /World/Machine/Frame | Xform | 2 | scenes/cutting_world.usda:370; scenes/cutting_world_user_edit_backup.usda:508 |
| /World/Machine/Frame/LeftFrontPost | Cube | 1 | scenes/cutting_world.usda:372 |
| /World/Machine/Frame/LeftPost | Cube | 1 | scenes/cutting_world_user_edit_backup.usda:510 |
| /World/Machine/Frame/LeftRearPost | Cube | 1 | scenes/cutting_world.usda:383 |
| /World/Machine/Frame/RightFrontPost | Cube | 1 | scenes/cutting_world.usda:405 |
| /World/Machine/Frame/RightPost | Cube | 1 | scenes/cutting_world_user_edit_backup.usda:521 |
| /World/Machine/Frame/RightRearPost | Cube | 1 | scenes/cutting_world.usda:416 |
| /World/Machine/Frame/YRailLeft | Cube | 2 | scenes/cutting_world.usda:394; scenes/cutting_world_user_edit_backup.usda:532 |
| /World/Machine/Frame/YRailRight | Cube | 2 | scenes/cutting_world.usda:427; scenes/cutting_world_user_edit_backup.usda:543 |
| /World/Machine/Kinematics | Xform | 2 | scenes/cutting_world.usda:439; scenes/cutting_world_user_edit_backup.usda:565 |
| /World/Machine/Kinematics/YBridge | Xform | 2 | scenes/cutting_world.usda:441; scenes/cutting_world_user_edit_backup.usda:567 |
| /World/Machine/Kinematics/YBridge/CrossBeam | Cube | 2 | scenes/cutting_world.usda:808; scenes/cutting_world_user_edit_backup.usda:934 |
| /World/Machine/Kinematics/YBridge/LeftYCarriage | Cube | 1 | scenes/cutting_world.usda:819 |
| /World/Machine/Kinematics/YBridge/LeftYServo | Cylinder | 1 | scenes/cutting_world.usda:841 |
| /World/Machine/Kinematics/YBridge/RightYCarriage | Cube | 1 | scenes/cutting_world.usda:830 |
| /World/Machine/Kinematics/YBridge/RightYServo | Cylinder | 1 | scenes/cutting_world.usda:854 |
| /World/Machine/Kinematics/YBridge/XCarriage | Xform | 2 | scenes/cutting_world.usda:891; scenes/cutting_world_user_edit_backup.usda:945 |
| /World/Machine/Kinematics/YBridge/XCarriage/CarriageBlock | Cube | 2 | scenes/cutting_world.usda:1258; scenes/cutting_world_user_edit_backup.usda:1312 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZBallScrew | Cylinder | 1 | scenes/cutting_world.usda:1282 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZServo | Cylinder | 1 | scenes/cutting_world.usda:1269 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide | Xform | 2 | scenes/cutting_world.usda:1295; scenes/cutting_world_user_edit_backup.usda:1323 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead | Xform | 2 | scenes/cutting_world.usda:1673; scenes/cutting_world_user_edit_backup.usda:1701 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/Blade | Mesh | 1 | scenes/cutting_world_user_edit_backup.usda:2081 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/BladeAxle | Cylinder | 1 | scenes/cutting_world.usda:2088 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/BladeForkLeft | Cube | 1 | scenes/cutting_world.usda:2066 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/BladeForkRight | Cube | 1 | scenes/cutting_world.usda:2077 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/BladeRotor | Xform | 1 | scenes/cutting_world.usda:2101 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/BladeRotor/Blade | Mesh | 1 | scenes/cutting_world.usda:2469 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/Body | Cylinder | 2 | scenes/cutting_world.usda:2053; scenes/cutting_world_user_edit_backup.usda:2068 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/CServo | Cylinder | 1 | scenes/cutting_world.usda:2040 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/CameraArm | Cube | 1 | scenes/cutting_world.usda:4397 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/LeftRoller | Cylinder | 2 | scenes/cutting_world.usda:2503; scenes/cutting_world_user_edit_backup.usda:2456 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/LeftRollerSupport | Cube | 1 | scenes/cutting_world.usda:2481 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/LineCamera | Xform | 1 | scenes/cutting_world.usda:2529 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/LineCamera/Housing | Cube | 1 | scenes/cutting_world.usda:2535 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/LineCamera/Lens | Cylinder | 1 | scenes/cutting_world.usda:2546 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/LineCamera/Sensor | Camera | 1 | scenes/cutting_world.usda:2559 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/RightRoller | Cylinder | 2 | scenes/cutting_world.usda:2516; scenes/cutting_world_user_edit_backup.usda:2469 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/CHead/RightRollerSupport | Cube | 1 | scenes/cutting_world.usda:2492 |
| /World/Machine/Kinematics/YBridge/XCarriage/ZSlide/Ram | Cube | 2 | scenes/cutting_world.usda:1662; scenes/cutting_world_user_edit_backup.usda:1690 |
| /World/Machine/Kinematics/YBridge/XLinearRail | Cube | 1 | scenes/cutting_world.usda:867 |
| /World/Machine/Kinematics/YBridge/XServo | Cylinder | 1 | scenes/cutting_world.usda:878 |
| /World/Machine/Vision | Xform | 1 | scenes/cutting_world_user_edit_backup.usda:555 |
| /World/Machine/Vision/IndustrialCamera | Xform | 1 | scenes/cutting_world_user_edit_backup.usda:557 |
| /World/MachineBlue | Material | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:102; scenes/fabric_feed_before_compliant_promotion.usda:102; +14 file |
| /World/MachineBlue/Shader | Shader | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:106; scenes/fabric_feed_before_compliant_promotion.usda:106; +14 file |
| /World/OmniverseGlobalRenderSettings | RenderSettings | 18 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:16696; scenes/cutting_world_user_edit_backup.usda:2568; +16 file |
| /World/OmniverseGlobalRenderSettings/LdrColor | RenderVar | 18 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:16706; scenes/cutting_world_user_edit_backup.usda:2578; +16 file |
| /World/Orange | Material | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:132; scenes/fabric_feed_before_compliant_promotion.usda:132; +14 file |
| /World/Orange/Shader | Shader | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:136; scenes/fabric_feed_before_compliant_promotion.usda:136; +14 file |
| /World/PhysicsScene | PhysicsScene | 2 | scenes/physx_felt_only_test.usda:16761; scenes/physx_felt_test.usda:17216 |
| /World/PhysicsScene/omni_kit_widget_viewport_ViewportTexture_0 | RenderProduct | 2 | scenes/physx_felt_only_test.usda:16788; scenes/physx_felt_test.usda:17243 |
| /World/Post0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:324 |
| /World/Post2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:346 |
| /World/ReferenceFelt | Material | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:193; scenes/fabric_feed_before_straightening_rollers.usda:193; +10 file |
| /World/ReferenceFelt/Shader | Shader | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:197; scenes/fabric_feed_before_straightening_rollers.usda:197; +10 file |
| /World/ReferenceFelt/Texture | Shader | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:215; scenes/fabric_feed_before_straightening_rollers.usda:215; +10 file |
| /World/ReferenceFelt/UV | Shader | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:208; scenes/fabric_feed_before_straightening_rollers.usda:208; +10 file |
| /World/RimX0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:277 |
| /World/RimX2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:299 |
| /World/RimY0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:288 |
| /World/RimY2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:310 |
| /World/Room | Xform | 3 | isaac_projects/scenes/jetson_light_world.usda:9; scenes/jetson_light_world.usda:9; +1 file |
| /World/Room/Floor | Cube | 3 | isaac_projects/scenes/jetson_light_world.usda:11; scenes/jetson_light_world.usda:11; +1 file |
| /World/Room/Wall_Back | Cube | 3 | isaac_projects/scenes/jetson_light_world.usda:20; scenes/jetson_light_world.usda:20; +1 file |
| /World/Room/Wall_Left | Cube | 3 | isaac_projects/scenes/jetson_light_world.usda:29; scenes/jetson_light_world.usda:29; +1 file |
| /World/Room/Wall_Right | Cube | 3 | isaac_projects/scenes/jetson_light_world.usda:38; scenes/jetson_light_world.usda:38; +1 file |
| /World/Rubber | Material | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:147; scenes/fabric_feed_before_compliant_promotion.usda:147; +13 file |
| /World/Rubber/Shader | Shader | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:151; scenes/fabric_feed_before_compliant_promotion.usda:151; +13 file |
| /World/SacrificialMat | Material | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:72; scenes/fabric_feed_before_compliant_promotion.usda:72; +13 file |
| /World/SacrificialMat/Shader | Shader | 15 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:76; scenes/fabric_feed_before_compliant_promotion.usda:76; +13 file |
| /World/SacrificialTop | Cube | 3 | scenes/fabric_feed_before_compliant_promotion.usda:219; scenes/fabric_feed_before_compliant_v2.usda:219; +1 file |
| /World/SacrificialTop | Mesh | 12 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:2494; scenes/fabric_feed_before_straightening_rollers.usda:2494; +10 file |
| /World/SacrificialTop/Mat | Material | 2 | scenes/physx_felt_only_test.usda:2514; scenes/physx_felt_test.usda:2514 |
| /World/Sensors | Xform | 3 | isaac_projects/scenes/jetson_light_world.usda:79; scenes/jetson_light_world.usda:79; +1 file |
| /World/Sensors/LightSensor | Cube | 3 | isaac_projects/scenes/jetson_light_world.usda:81; scenes/jetson_light_world.usda:81; +1 file |
| /World/Sensors/LightSensor/omni_kit_widget_viewport_ViewportTexture_0 | RenderProduct | 1 | scenes/world.usda:107 |
| /World/StationBackplate | Cube | 1 | scenes/fixed_head_cutting_world.usda:379 |
| /World/Steel | Material | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:87; scenes/fabric_feed_before_compliant_promotion.usda:87; +14 file |
| /World/Steel/Shader | Shader | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:91; scenes/fabric_feed_before_compliant_promotion.usda:91; +14 file |
| /World/Table | Cube | 18 | isaac_projects/scenes/jetson_light_world.usda:48; scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:238; +16 file |
| /World/Table | Material | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:57; scenes/fabric_feed_before_compliant_promotion.usda:57; +14 file |
| /World/Table/Shader | Shader | 16 | scenes/backups/fabric_feed_cutting_world_before_manual_save_20261022_152414.usda:61; scenes/fabric_feed_before_compliant_promotion.usda:61; +14 file |
| /World/TableLeg0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:233 |
| /World/TableLeg1 | Cube | 1 | scenes/fixed_head_cutting_world.usda:244 |
| /World/TableLeg2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:255 |
| /World/TableLeg3 | Cube | 1 | scenes/fixed_head_cutting_world.usda:266 |
| /World/TableTop | Cube | 1 | scenes/fixed_head_cutting_world.usda:222 |
| /World/Tire | Material | 1 | scenes/fixed_head_cutting_world.usda:177 |
| /World/Tire/Shader | Shader | 1 | scenes/fixed_head_cutting_world.usda:181 |
| /World/Workcell | Xform | 2 | scenes/cutting_world.usda:137; scenes/cutting_world_user_edit_backup.usda:238 |
| /World/Workcell/FixtureBase | Cube | 2 | scenes/cutting_world.usda:356; scenes/cutting_world_user_edit_backup.usda:494 |
| /World/Workcell/Floor | Cube | 2 | scenes/cutting_world.usda:334; scenes/cutting_world_user_edit_backup.usda:472 |
| /World/Workcell/PathPreview | Xform | 1 | scenes/cutting_world_user_edit_backup.usda:441 |
| /World/Workcell/PathPreview/GreenSegments | BasisCurves | 1 | scenes/cutting_world_user_edit_backup.usda:443 |
| /World/Workcell/PathPreview/YellowSegments | BasisCurves | 1 | scenes/cutting_world_user_edit_backup.usda:457 |
| /World/Workcell/Table | Cube | 2 | scenes/cutting_world.usda:345; scenes/cutting_world_user_edit_backup.usda:483 |
| /World/Workcell/Workpiece | Xform | 2 | scenes/cutting_world.usda:139; scenes/cutting_world_user_edit_backup.usda:240 |
| /World/Workcell/Workpiece/FeltBlank | Mesh | 2 | scenes/cutting_world.usda:300; scenes/cutting_world_user_edit_backup.usda:401 |
| /World/Workcell/Workpiece/HoleMarkers | Xform | 2 | scenes/cutting_world.usda:141; scenes/cutting_world_user_edit_backup.usda:242 |
| /World/Workcell/Workpiece/HoleMarkers/Hole_00 | Cylinder | 2 | scenes/cutting_world.usda:143; scenes/cutting_world_user_edit_backup.usda:244 |
| /World/Workcell/Workpiece/HoleMarkers/Hole_01 | Cylinder | 2 | scenes/cutting_world.usda:156; scenes/cutting_world_user_edit_backup.usda:257 |
| /World/Workcell/Workpiece/HoleMarkers/Hole_02 | Cylinder | 2 | scenes/cutting_world.usda:169; scenes/cutting_world_user_edit_backup.usda:270 |
| /World/Workcell/Workpiece/HoleMarkers/Hole_03 | Cylinder | 2 | scenes/cutting_world.usda:182; scenes/cutting_world_user_edit_backup.usda:283 |
| /World/Workcell/Workpiece/HoleMarkers/Hole_04 | Cylinder | 2 | scenes/cutting_world.usda:195; scenes/cutting_world_user_edit_backup.usda:296 |
| /World/Workcell/Workpiece/HoleMarkers/Hole_05 | Cylinder | 2 | scenes/cutting_world.usda:208; scenes/cutting_world_user_edit_backup.usda:309 |
| /World/Workcell/Workpiece/HoleMarkers/Hole_06 | Cylinder | 2 | scenes/cutting_world.usda:221; scenes/cutting_world_user_edit_backup.usda:322 |
| /World/Workcell/Workpiece/HoleMarkers/Hole_07 | Cylinder | 2 | scenes/cutting_world.usda:234; scenes/cutting_world_user_edit_backup.usda:335 |
| /World/Workcell/Workpiece/HoleMarkers/Hole_08 | Cylinder | 2 | scenes/cutting_world.usda:247; scenes/cutting_world_user_edit_backup.usda:348 |
| /World/Workcell/Workpiece/HoleMarkers/Hole_09 | Cylinder | 2 | scenes/cutting_world.usda:260; scenes/cutting_world_user_edit_backup.usda:361 |
| /World/Workcell/Workpiece/HoleMarkers/Hole_10 | Cylinder | 2 | scenes/cutting_world.usda:273; scenes/cutting_world_user_edit_backup.usda:374 |
| /World/Workcell/Workpiece/HoleMarkers/Hole_11 | Cylinder | 2 | scenes/cutting_world.usda:286; scenes/cutting_world_user_edit_backup.usda:387 |
| /World/Workcell/Workpiece/RidgeInner | BasisCurves | 1 | scenes/cutting_world_user_edit_backup.usda:412 |
| /World/Workcell/Workpiece/RidgeInner | Mesh | 1 | scenes/cutting_world.usda:311 |
| /World/Workcell/Workpiece/RidgeOuter | BasisCurves | 1 | scenes/cutting_world_user_edit_backup.usda:426 |
| /World/Workcell/Workpiece/RidgeOuter | Mesh | 1 | scenes/cutting_world.usda:322 |
| /World/ZBallScrew | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:412 |
| /World/ZBottomBearing | Cube | 1 | scenes/fixed_head_cutting_world.usda:460 |
| /World/ZCoupling | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:436 |
| /World/ZRail0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:390 |
| /World/ZRail2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:401 |
| /World/ZServo | Cube | 1 | scenes/fixed_head_cutting_world.usda:425 |
| /World/ZSlide | Xform | 1 | scenes/fixed_head_cutting_world.usda:471 |
| /World/ZSlide/CarriagePlate | Cube | 1 | scenes/fixed_head_cutting_world.usda:4837 |
| /World/ZSlide/GuideBlock0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:4815 |
| /World/ZSlide/GuideBlock2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:4826 |
| /World/ZSlide/ScrewNut | Cube | 1 | scenes/fixed_head_cutting_world.usda:4848 |
| /World/ZSlide/ScrewNut/Blade | Mesh | 1 | scenes/fixed_head_cutting_world.usda:4933 |
| /World/ZSlide/ScrewNut/BladeChuck | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:4920 |
| /World/ZSlide/ScrewNut/Bridge0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:4974 |
| /World/ZSlide/ScrewNut/Bridge2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:5009 |
| /World/ZSlide/ScrewNut/EccentricGearcase | Cube | 1 | scenes/fixed_head_cutting_world.usda:4909 |
| /World/ZSlide/ScrewNut/Guide0 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:4961 |
| /World/ZSlide/ScrewNut/Guide2 | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:4996 |
| /World/ZSlide/ScrewNut/MainBody | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:4872 |
| /World/ZSlide/ScrewNut/Mount | Cube | 1 | scenes/fixed_head_cutting_world.usda:4861 |
| /World/ZSlide/ScrewNut/OscillationMotor200W | Cube | 1 | scenes/fixed_head_cutting_world.usda:4898 |
| /World/ZSlide/ScrewNut/Pad0 | Cube | 1 | scenes/fixed_head_cutting_world.usda:4949 |
| /World/ZSlide/ScrewNut/Pad2 | Cube | 1 | scenes/fixed_head_cutting_world.usda:4985 |
| /World/ZSlide/ScrewNut/TangentialLockCollar | Cylinder | 1 | scenes/fixed_head_cutting_world.usda:4885 |
| /World/ZTopBearing | Cube | 1 | scenes/fixed_head_cutting_world.usda:449 |

| USDC binary Prim xác nhận | Nguồn xác nhận |
| --- | --- |
| /Cloth | 17_rebuild_compliant_felt.py dòng 539 |
| /Cloth/FinishedPart | 17_rebuild_compliant_felt.py dòng 541–545; script 21 dòng 7 |
| /Cloth/Offcut | 17_rebuild_compliant_felt.py dòng 541–545 |
| /Sample | 17_rebuild_compliant_felt.py dòng 671–676 |

# Phụ lục C Class Function Index

| File | Class Function Method cấp module |
| --- | --- |
| artifacts/phan_tich_anh/render_phan_vung.py | polygon dòng 19, callout dòng 24 |
| isaac_projects/scripts/jetson_light/01_create_structure.py | resolve_stage_path dòng 16, create_stage dòng 64, main dòng 97 |
| isaac_projects/scripts/jetson_light/02_create_basic_geometry.py | resolve_stage_path dòng 25, require_prim dòng 43, set_transform dòng 51, set_display_color dòng 62, create_cube dòng 67, create_sphere dòng 84, create_sphere_light dòng 101, build_geometry dòng 119, main dòng 200 |
| isaac_projects/scripts/jetson_light/03_test_scene.py | resolve_stage_path dòng 33, main dòng 59 |
| scripts/02_create_prim_tree.py | print_prim_tree dòng 27, create_stage dòng 42, main dòng 56 |
| scripts/03_create_basic_geometry.py | require_prim dòng 12, set_transform dòng 18, set_display_color dòng 45, create_cube dòng 50, create_sphere dòng 66, build_scene dòng 82, main dòng 160 |
| scripts/cutting/01_create_cutting_scene.py | _set_transform dòng 33, _material dòng 53, _bind dòng 72, _cube dòng 76, _cylinder dòng 91, _sphere dòng 109, _hexagonal_rotary_blade dòng 123, _raised_ridge_mesh dòng 162, _curve dòng 208, _height_field_slab dòng 227, _create_visual_holes dòng 304, _unwrap_headings dòng 332, _lookahead_index dòng 341, _animate_cutting_head dòng 357, _create_machine dòng 491, _create_scene dòng 601, _write_outputs dòng 716, main dòng 757 |
| scripts/cutting/02_validate_cutting_scene.py | _require dòng 47, main dòng 52 |
| scripts/cutting/03_play_cutting_demo.py | play_cutting_demo dòng 13, pause_cutting_demo dòng 19, reset_cutting_demo dòng 24 |
| scripts/cutting/05_create_fixed_head_scene.py | attr dòng 30, cube dòng 36, cyl dòng 40, xf dòng 44, material dòng 50, label dòng 58, make_blade dòng 62, make_table dòng 80, make_carrier dòng 103, make_tool dòng 162, make_vision dòng 191, main dòng 241 |
| scripts/cutting/06_validate_fixed_head_scene.py | main dòng 14 |
| scripts/cutting/07_fixed_head_controls.py | FixedHeadControls dòng 10 |
| scripts/cutting/08_render_fixed_head.py | capture dòng 28, main dòng 44 |
| scripts/cutting/09_create_fabric_feed_scene.py | attr dòng 27, material dòng 33, cube dòng 41, cyl dòng 45, xform dòng 49, flat_ridge dòng 55, make_cell dòng 83, make_sheet dòng 106, make_omni_wheels dòng 124, make_knife dòng 173, make_vision dòng 200, main dòng 239 |
| scripts/cutting/10_validate_fabric_feed_scene.py | main dòng 17 |
| scripts/cutting/11_fabric_feed_controls.py | FabricFeedControls dòng 12 |
| scripts/cutting/12_render_fabric_feed.py | capture dòng 23, main dòng 36 |
| scripts/cutting/13_convert_to_two_stepper_wheels.py | custom dòng 32, make_wheel dòng 40, main dòng 103 |
| scripts/cutting/14_validate_two_stepper_wheels.py | main dòng 16 |
| scripts/cutting/15_upgrade_physical_cutting.py | custom dòng 36, bind dòng 47, rotate_xy dòng 51, fabric_surface_z dòng 56, update_deck_and_access_slots dòng 70, create_heightfield dòng 87, signed_area dòng 121, simplify_polygon dòng 126, point_in_triangle dòng 144, triangulate dòng 150, create_finished_part dòng 168, inside_polygon dòng 186, offset_polygon dòng 195, create_scrap dòng 210, update_surface_details dòng 248, local_surface_at_world dòng 263, make_spring dòng 268, make_pair dòng 296, main dòng 373 |
| scripts/cutting/16_validate_physical_cutting.py | close dòng 25, main dòng 29 |
| scripts/cutting/17_rebuild_compliant_felt.py | attr dòng 30, clear_animation dòng 36, translate dòng 43, fixed_body dòng 48, spring dòng 53, smooth_cylinder dòng 63, triangular_gusset dòng 98, pinch_contact_skin dòng 116, mechanism dòng 155, metal_guide_tip dòng 275, straightening_rollers dòng 328, mesh_prim dòng 454, texture_material dòng 473, main dòng 504 |
| scripts/cutting/18_render_compliant_felt.py | main dòng 19 |
| scripts/cutting/19_check_shell.py | Không định nghĩa class/function cấp module |
| scripts/cutting/20_validate_compliant_felt.py | visible_triangles dòng 47, row dòng 67, exact_rows dòng 74, prominence dòng 77, components dòng 426 |
| scripts/cutting/21_sync_cloth_cache_clock.py | Không định nghĩa class/function cấp module |
| scripts/cutting/22_finalize_candidate.py | set_attr dòng 12 |
| scripts/cutting/24_fill_large_table_openings.py | set_string dòng 18, restore_small_perforations dòng 25, make_continuous_deck dòng 47, main dòng 82 |
| scripts/cutting/25_compact_direct_wheel_mount.py | set_attr dòng 26, set_translate dòng 33, set_cube dòng 43, define_cube dòng 51, set_cylinder dòng 64, define_smooth_cylinder dòng 73, spring_points dòng 111, main dòng 123 |
| scripts/cutting/27_render_pinch_diagnostic.py | capture dòng 20 |
| scripts/cutting/28_scrub_candidate_animation.py | main dòng 9 |
| scripts/cutting/29_add_pinch_contact_skins.py | main dòng 13 |
| scripts/cutting/30_apply_pending_mechanism_layout.py | main dòng 15 |
| scripts/cutting/30_physx_felt_setup.py | layer_signature dòng 89, _stable dòng 103, changed_outside dòng 118, mesh_area dòng 130, world_scale dòng 145, shape_volume dòng 151, bound_look_name dòng 186, Scope dòng 195, ensure_physics_material dòng 221, bind_physics_material dòng 238, configure_collider dòng 245, configure_tools dòng 260, felt_photo_albedo dòng 292, felt_normal_map dòng 316, configure_felt_pbr dòng 328, create_felt_test_sheet dòng 378, configure_felt dòng 433, body_inertia_about_axis dòng 492, configure_joints dòng 514, create_physics_scene dòng 580, configure dòng 595, main dòng 621 |
| scripts/cutting/31_physx_felt_smoke_test.py | world_points dòng 47, world_yaw dòng 53 |
| scripts/cutting/test_path_planner.py | main dòng 17 |
| scripts/jetson_light/01_create_structure.py | create_stage dòng 51, main dòng 75 |
| scripts/jetson_light/02_create_basic_geometry.py | require_prim dòng 28, set_transform dòng 36, set_display_color dòng 63, create_cube dòng 68, create_sphere dòng 85, create_sphere_light dòng 102, build_geometry dòng 120, main dòng 201 |
| scripts/jetson_light/03_test_scene.py | main dòng 50 |
| scripts/jetson_light/04_test_light.py | read_intensity dòng 18, turn_light_on dòng 25, turn_light_off dòng 30, main dòng 35 |
| scripts/jetson_light/05_virtual_light_sensor.py | parse_args dòng 15, main dòng 26 |
| scripts/jetson_light/06_automatic_light_control.py | parse_args dòng 20, decide_light_state dòng 31, main dòng 36 |
| scripts/jetson_light/06_daylight_sensor_simulation.py | _get_or_create_float_attr dòng 45, _get_or_create_sun dòng 53, _daylight_from_hour dòng 73, _update_sensor_and_lights dòng 80, _daylight_loop dòng 135, set_simulated_time dòng 168, set_day_duration dòng 186, pause_daylight_cycle dòng 198, resume_daylight_cycle dòng 206, stop_daylight_sensor_simulation dòng 214, start_daylight_sensor_simulation dòng 223 |
| scripts/jetson_light/06_live_automatic_light_control.py | set_virtual_light_level dòng 23, _live_control_loop dòng 52, stop_live_light_controller dòng 102, start_live_light_controller dòng 111 |
| scripts/jetson_light/07_isaac_ros2_light_bridge.py | parse_args dòng 26, IsaacLightBridge dòng 37, main dòng 85 |
| scripts/jetson_light/08_jetson_light_controller.py | parse_args dòng 20, decide_light_command dòng 32, JetsonLightController dòng 37, main dòng 62 |
| scripts/open_stage.py | load_project_stage dòng 27 |
| src/cutting/__init__.py | Không định nghĩa class/function cấp module |
| src/cutting/fabric_feed_planner.py | FabricSample dòng 30, drive_direction dòng 47, _mapped_z dòng 57, plan_cycle dòng 76, validate_plan dòng 122 |
| src/cutting/felt_shell.py | contour dòng 96, holes dòng 113, inside dòng 126, outward_normals dòng 134, miter_distance dòng 141, rib_profile dòng 171, base_shape_z dòng 184, shape_z dòng 199, swept_contact_points dòng 205, build_mesh dòng 231, ShellSolver dòng 343 |
| src/cutting/fixed_head_planner.py | PathPoint dòng 29, Sample dòng 36, rotate dòng 53, unwrap dòng 58, outline dòng 62, heading dòng 102, ridge_paths dòng 106, _smooth dòng 122, plan_cycle dòng 126, body_displacement dòng 209, _wheel_commands dòng 217, validate_plan dòng 281 |
| src/cutting/path_planner.py | PlannerConfig dòng 16, CutPoint dòng 28, _catmull_rom dòng 76, sample_closed_spline dòng 101, surface_height dòng 121, _signed_area dòng 130, _unit_tangent dòng 138, offset_closed_path dòng 149, _curvature dòng 165, _confidence dòng 178, _speed_for_point dòng 190, _zone dòng 202, plan_cut_path dòng 210, closed_path_length dòng 290 |

# Phụ lục D Module Dependency Matrix

| Module | Phụ thuộc | Cung cấp | Consumer Output |
| --- | --- | --- | --- |
| path_planner | Không | planner tổng hợp | 01_create_cutting_scene; fixed_head_planner |
| fixed_head_planner | path_planner | state machine và IK 4 bánh | 05_create_fixed_head_scene; fabric_feed_planner; 17_rebuild |
| fabric_feed_planner | fixed_head_planner | map chuyển động carrier sang vải và wheel | 09_create_fabric_feed_scene |
| felt_shell | numpy scipy | mesh và ShellSolver | 17_rebuild; 19; 20 validation |
| 17_rebuild_compliant_felt | felt_shell fixed_head_planner pxr Pillow | bake cache compliant | USDA candidate; USDC; JSON/PNG texture |
| open_stage | omni isaacsim | mở stage và nạp UI | 11_fabric_feed_controls hoặc 07_fixed_head_controls |
| IsaacLightBridge | rclpy std_msgs pxr | ROS bridge đèn | world.usda |
| JetsonLightController | rclpy std_msgs | quyết định threshold ngoài Isaac | IsaacLightBridge |

# Phụ lục E Cấu hình và tài nguyên ngoài

| Tên | Ý nghĩa | Nguồn |
| --- | --- | --- |
| ghrc_2026:v0 | Docker image Isaac Sim; không nằm trong repository | start_isaac.sh |
| /isaac-sim | Isaac Sim trong container | container_init.sh |
| /workspace/He_Thong_Cat_Ni_3D | Mount path cố định trong container | launcher và nhiều hướng dẫn |
| NVIDIA driver/container toolkit | Cấp GPU cho Docker | suy ra trực tiếp từ --gpus all; phiên bản không xác định |
| ROS 2 network | Jetson và Isaac discovery | ROS_DOMAIN_ID và network host; chưa nghiệm thu |
| Ảnh mẫu nỉ thật | artifacts/phan_tich_anh/mau_vai_ghe_goc.jpg | Nguồn hình học chỉ là ảnh, không phải scan 3D |

# Phụ lục F Bằng chứng đường dẫn và số dòng

| Kết luận | Bằng chứng |
| --- | --- |
| Stage mặc định | scripts/open_stage.py:17–23; start_isaac.sh:125 |
| Tự nạp UI fabric feed | scripts/open_stage.py:54–63 |
| Planner synthetic | src/cutting/path_planner.py:50–73, 178–207, 210–287 |
| Bake wheel/Z/controller/cloth | scripts/cutting/17_rebuild_compliant_felt.py:577–669 |
| ROS topics/QoS | scripts/jetson_light/07_isaac_ros2_light_bridge.py:20–23, 37–82 |
| Jetson threshold | scripts/jetson_light/08_jetson_light_controller.py:32–59 |
| Live controller collision prevention một phần | 06_live_automatic_light_control.py:111–124; 06_daylight_sensor_simulation.py:223–256 |
| Physics limitation | docs/PHYSX_FELT_SETUP.md:8–21, 183–189 |
| Namespace mismatch | scenes/fabric_feed_cutting_world.usda:2507 trở đi; relationship tại 2632–2641 và 2781–2782 |
| Candidate equals main | SHA-256 650a7f525167b2ca757cacbfc8704240a969317cd6bbe9fa9cba039a6b982b44 |

# Phụ lục G Thông tin chưa xác định

Không xác định lịch sử commit, branch, tác giả thay đổi hoặc task cuối vì .git không hợp lệ.

Không xác định chính xác NVIDIA driver, CUDA, Python ABI và dependency versions ngoài mô tả image.

Không đọc được nội dung ba core dump và một phần .isaac_cache do quyền truy cập.

Không xác định calibration camera thật, TF, kích thước máy thật, encoder, motor driver, PLC và e-stop.

Không có bằng chứng Jetson thật đã kết nối hoặc ROS 2 topic đã trao đổi trên mạng.

Không có bằng chứng scene main đã được kiểm tra lại sau khi namespace đổi từ FeedHead/CuttingHead/Vision sang AirTable.

Không xác định file Blender có phải nguồn hình học chính hay chỉ bản thử.

# Phụ lục H Prompt khởi tạo Codex trên máy mới

Bạn đang tiếp nhận dự án NVIDIA Isaac Sim tại project root đang mở. Trước khi làm bất kỳ thay đổi nào, hãy đọc toàn bộ tài liệu NVIDIA_ISAAC_SIM_COMPLETE_PROJECT_HANDOVER.docx và đối chiếu mọi kết luận với repository hiện tại.

Hãy xác định project root thực tế, kiểm kê file và kiểm tra stage mặc định từ start_isaac.sh cùng scripts/open_stage.py. Stage mặc định tại thời điểm bàn giao là scenes/fabric_feed_cutting_world.usda, nhưng bạn không được giả định điều này nếu code đã đổi. Đặc biệt kiểm tra namespace Prim: tài liệu phát hiện scene hiện tại dùng /World/AirTable trong khi nhiều script và relationship cũ dùng /World/FeedHead, /World/CuttingHead và /World/Vision.

Phân biệt rõ bốn cơ chế: script tạo hoặc nâng cấp scene; planner offline; USD/USDC time-sample playback; và controller runtime. Không gọi time samples đã bake là controller hardware. Không coi custom vision attributes là kết quả camera thật. Giữ nguyên nhãn synthetic, assumed, provisional, uncalibrated và unverified.

Không sửa file, scene, kiến trúc hoặc tạo adapter nếu người dùng chưa yêu cầu cụ thể. Không chạy script có thể Save/Export hoặc tạo output chỉ để kiểm tra. Không tự ý triển khai Jetson, ROS 2, vision, safety hoặc machine control. Khi có yêu cầu tích hợp Jetson, trước hết lập lại ownership matrix cho từng Prim/attribute và bảo đảm chỉ một writer. Không cho Jetson ghi trực tiếp vào attribute đang có time samples hoặc PhysX drive nếu chưa có external-control mode, arbitration, watchdog và interlock.

Khi tài liệu khác code, ưu tiên kiểm tra code và scene thực tế, báo cáo sai khác kèm file, function và dòng. Nếu cần tiếp tục phát triển, trình bày phạm vi, rủi ro và file dự kiến thay đổi rồi chờ yêu cầu cụ thể của người dùng.

# Kiểm tra chất lượng và giới hạn kiểm toán

| Hạng mục | Kết quả |
| --- | --- |
| Project root | Đã xác nhận |
| Kiểm kê file ngoài cache | Đã hoàn thành |
| File không đọc được | Ba core dump và một phần .isaac_cache |
| USD hierarchy | Đã parse toàn bộ USDA; USDC qua metadata/code/reference |
| Interaction/data flow | Đã mô tả theo bằng chứng |
| Control ownership Jetson | Đã lập bảng bắt buộc |
| Runtime execution | Không chạy để giữ read-only |
| Git history | Không thể vì .git không hợp lệ |
| Word render | Đã render và kiểm tra trực quan toàn bộ 98 trang; không có clipping, overlap hoặc bảng hỏng |